CP/M-80 KERMIT VERSION 4.11 USER GUIDE

C. Gianone

Columbia University Center for Computing Activities

New York, New York 10027

April 23, 1991

Copyright (C) 1981,1991

Trustees of Columbia University in the City of New York

Permission is granted to any individual or institution to use, copy,

or redistribute this document so long as it is not sold for profit, and

provided this copyright notice is retained.

1. CP/M-80 KERMIT

********

This document is formatted as an ordinary, plain text ASCII disk file.  Typeset

copies are available in the Kermit User Guide from Columbia University.

Changes should be made to CPKERM.MSS.

********

Program:    Mike Freeman, Bonneville Power Administration, Vancouver, WA, USA,

with contributions from many others.

Language:   8080 Assembler, LASM, M80, or MAC80

Version:    4.11

Date:       April 1, 1991

Documentation:  Christine Gianone, Columbia University, with contributions from

many others.

KERMIT-80 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Partial, Auto-receive only

Login scipts:                      Yes, limited

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     Yes

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            Yes

Terminal emulation:                Yes, VT52 and others

Communication settings:            Yes

Support for dial-out modems:       No

Transmit BREAK:                    Yes; most versions

IBM communication:                 Yes

Transaction logging:               No

Debug logging:                     No

Session logging:                   Yes

Raw file transmit:                 Yes

Act as server:                     No

Talk to server:                    Yes

Advanced commands for servers:     Yes

Command/init files:                Yes

Command macros:                    No

Local file management:             Yes

Handle file attributes:            No

Long packets:                      No

International Character Sets:      No

Sliding Windows:                   No

Printer control:                   Yes, limited

1.1. Credits

CP/M Kermit is the first of all the Kermit programs.  It was originally written

by Bill Catchings of Columbia University in 1981.  Over the years,

contributions have been added by many people, including Charles Carvalho (ACC),

Bernie Eiben (DEC), Nick Bush (Stevens Institute of Technology), John Bray

(University of Tennessee), Bruce Tanner (Cerritos College), Greg Small

(University of California at Berkeley), Kimmo Laaksonen (Helskini University of

Technology), Bertil Schou (Loughborough University), Jon Warbrick (Plymouth

Polytechnic University), Brian Robertson (Aberdeen University), A.J. Cole

(Leeds University), John Shearwood (Birmingham University), Tony Addyman

(Salford University), Godfrey Nix and Martin Carter (Nottingham University),

Ian Young (Edinburgh University), Chris Miles (Manchester University), Richard

Russell, Dave Roberts, and many, many others.

Version 4.11 is the work of Mike Freeman of the Bonneville Power Administration

in Vancouver, WA, USA, with assistance from Russell Lang of Monash University

in Australia, Jay S Rouman of Mt Pleasant MI, and others.

1.2. What's New

Features added since version 4.09 include:

- SET COLLISION {BACKUP/DISCARD/OVERWRITE/RENAME}

- SET INCOMPLETE-FILES {DISCARD/KEEP}

- Many REMOTE commands, including some REMOTE SET commands

- RENAME command to rename CP/M files from within Kermit-80

- SET RECEIVE/SEND PACKET-LENGTH nn (nn <= 94)

- SET AUTORECEIVE ON now implies that Kermit-80 ALWAYS tries to receive

more files when a RECEIVE transaction has completed.  The user can

cancel with ^C.

- QUIT is now a synonym for EXIT.

- STAY is now a synonym for SET NO-EXIT.

- CONNECT, RECEIVE and SEND may be abbreviated to C, R and S,

respectively.

- Cancellation of TAKE, TYPE, and PRINT commands from the keyboard.

- Many bug fixes.

- Kermit-80 Version 4.11 now supports the Microbee family of computers

(56K, 64K, 128K and 256K) manufactured by Microbee Systems, Ltd, of

Australia.

- Kermit-80 now supports the Ampro Little Board system.

1.3. Overview of Kermit Operation

Use the SET command to establish necessary communication parameters like SPEED

and PARITY.  Use the CONNECT to establish a terminal connection to the remote

computer.  If you are dialing out with a modem, type the necessary dialing

commands to the modem first.  The dialing process can be automated to some

extent using a TAKE command file containing INPUT, OUTPUT, and PAUSE commands.

Then log in to the remote computer or service and conduct a session.

To transfer a text file, start the Kermit program on the remote computer and

tell it to SEND the desired file (if uploading) or to RECEIVE (if downloading).

"Escape back" to CP/M Kermit, usually by typing Ctrl-] (hold down the Control

key and press the right bracket key) and then type the letter C. At the CP/M

Kermit prompt type RECEIVE (if you gave a SEND command to the remote Kermit) or

SEND filename (if you gave a receive command to the remote Kermit).

To transfer a binary file, give the command SET FILE TYPE BINARY to the remote

Kermit and SET FILE-MODE BINARY to CP/M Kermit before issuing any SEND or

RECEIVE commands.

Multiple files of the same type (text or binary) can be transferred in a single

operation using "wildcard notation" (including special characters like asterisk

in the filename).

When file transfer is complete, CONNECT back to the remote computer, use the

EXIT command to exit from the remote Kermit program, finish your work on the

remote computer, log out from it, escape back to CP/M Kermit again, and EXIT

from CP/M Kermit.

The remote Kermit may also be put into "server mode" to simplify these

operations.  Give the SERVER command to the remote Kermit, escape back to CP/M

Kermit, and then issue SEND commands to send files (upload), GET filename

commands to receive (download) files, REMOTE commands to request various other

services (like directory listings) from the remote Kermit.  When you are done,

give a BYE command to terminate your remote session, or a FINISH command to

tell the remote Kermit to return to its prompt so you can CONNECT back and

conduct further business.

That's all there is to it.

1.4. Summary of CP/M

There are essentially two versions of CP/M - Versions 2.2 and 3.0 (sometimes

also called CP/M PLUS.)

CP/M-80 Version 2.2 is run in a single 64 Kbyte "page", usually the largest

amount of memory on Z80 or 8080 systems.  The BIOS (Basic input/output system),

BDOS (Basic Disk Operating System) and CCP (Command console processor) all

share memory with any transient program the user may wish to run.  Some basic

commands are available through the CCP, like DIR, ERA etc,while others are

loaded from disk into the transient program area and run as a program, like PIP

or STAT.

CP/M Version 3.0 (or CP/M PLUS) effectively removes the requirement of having

the CCP and BDOS along with a chunk of the BIOS code being resident in the

single 64k byte page of memory.  This allows even more space for programs in

the TPA, but still a little less than the maximum of 64k.  It is substantially

different from CP/M version 2.2, with lots of added features.  Kermit-80 uses

very few additional version 3.0 features, and only where absolutely necessary.

CP/M file specifications are of the form DEV:XXXXXXXX.YYY, where

DEV:            is a device name, normally the A: or B: floppy.  If omitted,

the device name defaults to your connected diskette.

XXXXXXXX        is a filename of up to 8 characters.

YYY             is the file type, up to 3 characters.

File names and file types may contain letters, digits, and some special

characters, including dash, dollar sign, and underscore, but no imbedded

spaces.  Upper and lower case letters are equivalent.

"Wildcard" file-group specifications are permitted in file names and file types

(but not device names) within certain contexts; a "*" matches a whole field, a

"?"  matches a single character, including space.  Examples: "*.F??" specifies

all files whose types start with F and are 1, 2, or 3 characters long; "F?.*"

specifies all files whose names start with F and are no more than two

characters long (before the trailing spaces).

The five CP/M commands are:

DIR file        Lists the the names of the specified files.  The default file

specification is "*.*".  Example: "DIR B:*.FOR".

ERA file        Erases (deletes) the specified file(s); wildcards allowed.

REN new old     Changes the name of a file from old to new, e.g.

"REN NEW.FOR=OLD.FOR".

SAVE            Saves the specified number of memory blocks into a file.  (Not

on CP/M Plus systems)

TYPE file       Types the specified file on the screen, e.g.  "TYPE FOO.TXT".

The most important programs are:

STAT            Gives statistics on disk usage; sets and displays IOBYTE.  (Not

on CP/M Plus systems)

PIP             Peripheral Interchange Program.  Copies files.  In response to

the "*" prompt, give a command of the form

disk:outfile=disk:infile

Wildcards ("*" for a whole field or "?"  for a letter) can be

used.  Examples: "A:=B:*.*" to copy a whole disk, "A:=B:*.FOR"

to copy all the Fortran programs from disk B to disk A. If the

disk specification is omitted, your "connected" disk is

assumed.  Command line arguments are also accepted, e.g. "PIP

A:=B:*.*".

There are equivalent commands for CP/M Version 3.0, but are not loaded into

memory in the same way as for CP/M Version 2.2.  For further information on

CP/M, consult your microcomputer manual or a CP/M handbook.

1.5. Kermit-80 Description

Since Kermit-80 runs on a standalone micro, it is always in control of the

screen -- it is always in "local mode".  It includes a terminal emulator for

establishing a connection to a remote computer or service, and during file

transfer, it keeps the screen updated with the file name and the packet number,

whether sending or receiving.

Kermit-80 is capable of an imprecise or "fuzzy" timeout on an input request,

and can break deadlocks automatically.  In most cases, this is not important,

because the Kermit program on the other side is most likely able to handle the

timeouts.  The timeouts done by Kermit-80 are fuzzy because they depend on the

speed of the processor and other factors that can vary from system to system.

If, despite the timeout capability, the transmission appears to be stuck (and

you can tell that this has happened if the screen fails to change for a while)

you can type carriage return to have the micro do what it would have done on a

timeout, namely NAK the expected packet to cause theforeign host to send it

again (or, if the micro is sending, to retransmit the last packet).  Micro/

micro or micro/IBM-mainframe transfers could require this kind of manual

intervention.

File transfers may be interrupted in several ways.

Control-C       This will return you to Kermit-80 command level immediately, so

that you can connect back to the remote system, or take any

other desired action.

Control-X       When sending a file, this will terminate the sending of the

current file with a signal to the KERMIT on the other side to

discard what it got so far.  If there are more files to be

sent, KERMIT-80 will go on to the next one.  When receiving a

file, KERMIT-80 will send a signal to the remote KERMIT to stop

sending this file.  If the remote KERMIT understands this

signal (not all implementations of KERMIT do), it will comply,

otherwise the file will keep coming.  In any case, the remote

KERMIT will go on to the next file in the group, if any.

Control-Z       Like Control-X, except if a file group is being transmitted,

this will stop the transmission of the entire group.  If only a

single file is being transmitted, it works exactly like

Control-X.

Carriage Return If you type a carriage return Kermit-80 will resend the current

packet.  You may do this repeatedly, up to the packet retry

limit (somewhere between 5 and 16 times) for a particular

packet.

KERMIT-80 COMMANDS

Kermit-80 is an interactive program.  It issues a prompt, you type a command.

The process repeats until you give the EXIT command to leave the program.

Commands consist of keywords, filenames, and numbers.  Keywords may be

abbreviated to minumum unique length.  "?" may be typed to request a menu of

the available options for the current field at any point in a command.  ESC may

be typed at any point in a command to fill out the current keyword or filename;

if sufficient characters have not been typed to identify the current field

uniquely, Kermit-80 will sound a beep and allow you to continue from that

point.  Here are Kermit-80's commands:

BREAK   Send a BREAK condition to the remote computer.  This is only possible

if your system is capable of sending breaks.  It is intended to be used

with PAUSE, OUTPUT, etc and the TAKE command to do wierd and wonderful

things, like automatic logging on to a remote host.

BYE     When talking to a remote Kermit Server, this command shuts down the

server and logs it out, and also exits from Kermit-80 to CP/M command

level.

CONNECT Establish a terminal connection to the computer, service, or device

that is connected to the serial port, i.e. pass all typein to the

serial port and display all input from the serial port on the screen.

Also, emulate a DEC VT52 to allow cursor control, screen clearing,

etc., if VT52-EMULATION is ON (see below), in which case you should

also set your terminal type on the remote host to VT52.  (Some versions

emulate other terminals.)  The CONNECT command may be abbreviated by

the single letter C.

Warning: VT52 emulation is only successful if your system or its

attached terminal can do the same sort of functions as a genuine VT52.

Things to beware of are cursor addressing, clear to end of page and end

of line, clear screen, home cursor, and clear-and-home functions.  The

useability of VT52 emulation depends entirely on how many of the VT52

functions can be emulated by your micro or terminal.

The escape character differs from micro to micro; when you issue the

CONNECT command, the micro will print a message telling you how to get

back.  The escape sequence is generally an uncommonly-used control

character, like CTRL-backslash or CTRL-rightbracket, followed by a

single letter "command":

C   Close Connection, return to Kermit-80> command level.

S   Display Status of connection, but maintain remote connection.

?   List available single-character commands.

0   (zero) Send a null (0) character.

B   Send a BREAK signal.  Most systems provide this function.

D   Drop the line.  Used on the Apple with modem.  Automatically closes

the connection after dropping the line.  The TORCH system

acknowledges this command but does nothing.

P   Toggle printer on or off.  Allows you to copy whatever goes to the

screen to the printer.

S   Temporarily suspend logging to the log file.

Q   Restart logging to the log file

^]  (or whatever - a second copy of the escape character) Send the

escape character itself to the remote host.

COPY source destination

Copy a named file to another file, either on the same drive or another

drive.

DIRECTORY

This provides a directory listing of the specified files.  If no files

are specified, all files on the default disk are listed.  File sizes,

in K, are included.  You may interrupt the listing at any time by

typing any character.  The listing (even if interrupted) concludes with

a display of the amount of free storage left on the disk.  You can

inhibit the display of file sizes by SET DIRECTORY OFF.

ERASE filespec

This executes the CP/M ERA command on the specified file(s).  The names

of the files being erased are not displayed.

EXIT    Quit back to CP/M.  The return is made by a JMP 0 (Warmstart).  QUIT is

a synonym for EXIT.

FINISH  Like LOGOUT, but shuts down the remote server without logging it out.

Leaves you at Kermit-80 command level; subsequent CONNECT commands will

put you back at host system command level.

GET filespec [local_filespec]

When Kermit-80 is talking to a Kermit Server on the host, you should

use the GET command to request the server to send files to you, for

example:

get hlp:k*.hlp

You may specify a local filename if you want to save the remote file

under a different filename.  Limitation: If you request an alternate

block check type using the SET BLOCK command, the GET command will not

communicate it to the remote server.  If you want to have type 2 or 3

block checks done when getting files from the server, you have to issue

the appropriate SET BLOCK command to the remote KERMIT before putting

it in server mode.

HELP    List all these commands, with a short description on what the commands

do.  A question mark will do the same.  If you have already typed a

command but do not know what the parameters are, type a space (to

indicate the end of the command) and a question mark.  You will be

informed of what Kermit can expect at that stage.

INPUT seconds text

Setup a text line and time delay for your CP/M system to expect from

the host, then wait up to the given number of seconds (approximately)

for text to be sent to your CP/M-80 system.

LOG filespec

When CONNECTed to a foreign host as a terminal, log the terminal

session to the specified diskette file.  This functionality depends to

some extent on the remote host's ability to do XON/XOFF flow control,

and does not guarantee a complete transcript (after all, that's what

the KERMIT protocol is for).  The log file is closed when the

connection is closed by typing the escape character followed by the

single-character command "C".

It is possible to temporarily suspend logging during connect state.

Typing an escape sequence can turn file logging on (<escape-character>

R for Resume) or off (<escape-character> Q for quiet).

Re-entering connect state will re-open the previously opened log file

and append to that file.

LOGOUT  Like BYE, but leaves you at Kermit-80 command level.

OUTPUT text

Send the text to the remote computer as if you had typed it.

PAUSE seconds

If this command is issued your CP/M system will wait a while before

proceeding with another command.  This is intended for use in TAKE

commands, where you may want to pause for a while before proceeding

with the rest of the TAKE file.  The actual delay is very variable

between systems, and values should be determined on a trial and error

basis.

PRINT   Print a file to the console and printer.  Output to the printer is

buffered by the Kermit-maintained printer buffer.  This routine is

identical to TYPE but characters are echoed to the printer as well as

to the screen.  Suspending and canceling output is as described in

TYPE.

QUIT    Synonym for EXIT.

RECEIVE filespec

Receive file(s) from the remote Kermit, and save them under the names

provided in the file headers supplied by the remote host.  If a local

filespec is given, the file is saved under the given filename.  If the

names aren't legal, use as many legal characters from the name as

possible (see the description of SET FILE-WARNING below).  If there's a

conflict, and FILE-WARNING is ON, warn the user and try to build a

unique name for the file by adding "&" characters to the name.  RECEIVE

can be abbreviated to the single letter R.

REMOTE command

Send a command to a remote Kermit server.  The results are sent back to

your CP/M screen.  When two arguments are required and specify less

than two in the command, you will be prompted for the missing

arguments.  REMOTE commands include:

REMOTE CD [directory]

Ask the remote server to change its default directory.  If no

directory is specified, the server changes to its login directory.

REMOTE COPY file1 file2

Ask the remote server to copy file1 to file2.

REMOTE RENAME file1 file2

Ask the remote server to rename file1 to file2.

REMOTE DELETE filespec

Ask the remote server to delete the named file or files.

REMOTE DIRECTORY [filespec]

Ask the remote server to display a directory listing of the given

files or, if the filespec is omitted, all the files in the current

device or directory.

REMOTE DISK-USAGE

Ask the remote server to display information about its disk usage

(such as free or used space).

REMOTE ERASE filespec

Same as REMOTE DELETE.

REMOTE FINISH

Same as FINISH.

REMOTE HELP

Ask the remote server to display a list of the commands it can

respond to.

REMOTE HOST command

Ask the remote server to have its operating system execute the

given command.

REMOTE KERMIT command

Ask the remote server to execute the given Kermit command, given in

the server Kermit's command syntax.

REMOTE LOGIN user password

Log in to a remote Kermit server which has been set up to require a

username and password.

REMOTE MESSAGE text

Send the text to the remote server for display on its screen

(useful with MS-DOS Kermit servers).

REMOTE SET parameter value

Ask the remote server to set the given parameter to the given

value, for example REMOTE SET FILE TYPE BINARY.  Type REMOTE SET ?

to see a list of the REMOTE SET options.

REMOTE SPACE

Same as REMOTE DISK-USAGE.

REMOTE STATUS

Ask the remote server to provide a status report.

REMOTE TYPE file

Ask the remote server to display the named file on the micro's

screen.

REMOTE WHO [user]

Ask the remote server for a list of users who are logged in, or if

a user is specified, for a report on the named user.

RENAME file1 file2

Rename local CP/M file1 to file2.

SEND filespec

Send file(s) specified by filespec to the remote Kermit.  The filespec

may contain CP/M wildcards.  SEND may be abbreviated to the single

letter S.

SET parameter [value]

Set the specified parameter to the specified value.  Possible parameter

settings:

AUTORECEIVE

ON (or OFF).  Allows several files to be received without

having to type RECEIVE on the receiving machine.  The routine

simply looks for activity on the serial line, and if so fudges

a RECEIVE command.  The packet sent by the sender will be lost.

BLOCK-CHECK-TYPE option

The options are:

1-CHARACTER-CHECKSUM

Normal, default, standard 6-bit checksum.

2-CHARACTER-CHECKSUM

A 12-bit checksum encoded as two characters.

3-CHARACTER-CRC-CCITT

A 16-bit CCITT-format Cyclic Redundancy Check, encoded

as 3 characters.

BUFFER-SIZE value

This allows you to set a buffer size during transfer of data.

On some systems it takes so long that the remote end times out

while the local system is reading or writing to disk.  The size

is the number of 128 disk sectors (nominal) and can be from 1

(128 bytes) to 64 (8 kbytes).

CP/M-80 filenames will still be mapped to uppercase characters.

COLLISION value

What to do when a file arrives that has the same name as an

existing file.  BACKUP means to rename the existing file.

DISCARD means to discard and reject the incoming file.

OVERWRITE means to overwrite the existing file.  RENAME means

to rename the existing file.

DEBUG   ON (or OFF).  Enables/disables displaying of packets on the

screen during file transfer.  Not performed if the QUIET option

has been set for the terminal (SET TERMINAL QUIET)

DEFAULT-DISK drive letter

This allows you to set the default disk as source and

destination of file transfers.  In addition, issuing this

command causes you to switch to the specified disk and log it

in, write-enabled.  The colon must be included in the disk name

(A:).  The selected disk appears in your KERMIT-80 prompt, for

instance

Kermit-80 14A:>

DIRECTORY-FILE-SIZE ON (or OFF).

By setting DIRECTORY-FILE-SIZE OFF you can get an abreviated

listing of your disk drive.  File sizes are not calculated, and

five files are shown on a line.  Setting this option ON will

show file sizes of each file.

Both options will list the free space remaining.

ESCAPE  Change the escape character for virtual terminal connections.

Kermit-80 will prompt you for the new escape character, which

you enter literally.

FILE-MODE option

Tells KERMIT-80 what kind of file it is sending, so that KERMIT

can correctly determine the end of the file.  SET FILE BINARY

means to send all the 128-byte blocks (ie logical CP/M sectors)

of the file, including the last block in its entirety; SET FILE

ASCII is used for text files, and transmission stops when the

first Control-Z is encountered anywhere in the file (this is

the CP/M convention for marking the end of a text file).

SET FILE-MODE DEFAULT tells Kermit to attempt to determine the

file type by examining the file being transmitted.  If a

Control-Z appears before the last block of the file, it is

assumed to be BINARY; if, when the first Control-Z is

encountered, the remainder of the file contains only

control-Z's, it is assumed to be a text file.  Unfortunately,

not all programs fill the remainder of the last record of a

text file with Control-Z's, so this algorithm is not always

successful.

If binary transmission is used on a text file, or a compressed

file (eg a .DQC file) some extraneous characters (up to 127 of

them) may appear at the end of the file on the target system.

If ASCII transmission is used on a binary file, any 8th bits

set will be stripped and a warning sent to the console.  When

the first control-Z is encountered, the file is assumed to be

at the end, even if it is not.

FLOW-CONTROL ON (or OFF)

Sets XON/XOFF flow control on or off.  If set ON the host is

expected to respond to an XOFF or XON sent by Kermit-80.  If

set off, no flow control is assumed and any XON/XOFF is

ignored.

IBM ON (or OFF)

Allow the transfer of files to and from an IBM mainframe

computer.  This makes Kermit-80 wait for the IBM turnaround

character (XON), ignore parity on input, add appropriate parity

to output, and use local echoing during CONNECT.  As

distributed, KERMIT-80 uses MARK parity for IBM communication.

If you don't give this command, IBM mode is OFF.  Since IBM

VM/CMS KERMIT does not have timeout capability, SET IBM ON also

turns on the "fuzzy timer" automatically.

LOCAL-ECHO ON (or OFF)

When you CONNECT to a remote host, you must set LOCAL-ECHO ON

if the host is half duplex, OFF if full duplex.  OFF by

default.

LOGGING ON (or OFF)

Cease or resume logging whenever connect mode is entered.  This

is really only applicable after a LOG command is no longer

required.

NO-EXIT This command is applicable only for Kermit initiated with a

command tail.  For example, if Kermit was initiated by:

KERMIT ;SEND HELLO;NO-EXIT

Kermit would first seek out and execute the KERMIT.INI file (if

present), then send file HELLO to a remote system.  Usually

Kermit would exit back to CP/M, but NO-EXIT over-rides this.

STAY is a synonym for NO-EXIT.

Note the leading semicolon.  This clears leading spaces from

the first command.

OUTPUT text-line

Send a line of text to the remote computer (or modem).  This

simply copies the string to the correct line, and assumes all

appropriate parameters have been set to be used, e.g. speed,

parity etc.  It is intended for use in TAKE command files.

PARITY option

Sets parity for outgoing characters to one of the following:

NONE, SPACE, MARK, EVEN, or ODD.  On input, if parity is NONE,

then the 8th bit is kept (as data), otherwise it is stripped

and ignored.  The parity setting applies to both terminal

connection and file transfer.  If you set parity to anything

other than none, KERMIT-80 will attempt to use "8th bit

prefixing" to transfer binary files.  If the other KERMIT is

also capable of 8th bit prefixing, then binary files can be

transferred successfully; if not, the 8th bit of each data byte

will be lost (you will see a warning on your screen if this

happens).

PORT port name

Allows you to switch between different communication ports.

This command is not available on all systems.  Type SET PORT ?

for a list of valid options for your system. (Note:  If your

system does not support several ports, this command will return

a "Not implemented" error if you try to set a port.)

PRINTER ON (or OFF)

Turns copying of CONNECT session to printer on and off.  It is

also possible to toggle the printer on/off from the connect

state, by typing <escape character> followed by P.

RECEIVE parameter [value]

Set a RECEIVE parameter.

PAD-CHAR

Set the PAD character to use while receiving files.

Currently a dummy, as for SET SEND PAD-CHAR.

PADDING [value]

Set the number of PAD characters to use while receiving

files.  Same as SET SEND PADDING.

START-OF-PACKET [value]

Set the default start of Packet character for receiving

files.  Apply the same rules and considerations as for

SET SEND START-OF-PACKET.

PACKET-LENGTH number

Tell the other Kermit the longest packet length CP/M

Kermit is willing to receive during file transfer.  The

maximum length is 94, which is also the default length.

SEND parameter [value]

Set a SEND parameter.

PAD-CHAR

Set the Pad character to be used while sending files.

It is currently a dummy entry, and does not do

anything.

PADDING [value]

Set the number of PAD-CHARS to be used while sending

files.  This too does nothing.

START-OF-PACKET

Set the default start of packet character to another

character than control-A.  This may be necessary on

systems (including intervening networks) that trap

control-A characters.  Choose a control character not

otherwise used, ie not carriage return (13D, ODH), line

feed (10D, OAN), tabs (09D, 09H), backspace (08H), and

bell (07H) or any other used between you and your

remote system.

SPEED value

Change the baud rate of the communications port.  This command

only works on some systems.  value is the numeric baud rate

(300, 9600, etc.) desired.  Type SET SPEED followed by a

question mark for a list of supported baud rates.  On systems

that do not support this command, you must set the port baud

rate from CP/M or other setup mechanism outside of KERMIT-80.

TACTRAP Set the TAC intercept character.  If you are attached to a TAC

it will swallow the intercept character (commercial AT sign by

default) so Kermit sends it twice.  With this command you can

set the intercept character (ie the one to send twice) to

another character.

TERMINAL option

Select one of the following terminal characteristics:

OFF     sets emulation off, and its up to the attached terminal

to respond to escape sequences sent from the remote

host system.

DUMB    Like off, but carriage return and line feed characters

are the only control characters accepted.  All other

control characters are simply ignored. (Really a "Glass

TTY").

EXTERNAL

Emulation is provided for by a routine in the system

dependent part of Kermit.  Attempting to set this

option without having and externally supplied routine

will returna "Not Implemented" error.

OFF     All characters are passed directly to the terminal

without any interpretation by Kermit.

VT52    When connected as a terminal to a foreign host, the

micro emulates a VT52.  VT52 emulation is set by

default, except on micros that already have terminal

functionality built in, such as the DEC VT180 and

DECmate (these act as VT100-series terminals).  Some

systems emulate other terminals, like the ADM3A; see

table 1-5.

QUIET   Do not display any file transfer information onto the

console.  This mode is useful if you console takes a

long time to update the display.  Only the file name is

displayed.  DEBUGging information is not displayed even

if selected.

REGULAR Inverse of QUIET.  All packets etc displayed, as

ususal.

TIMER ON (or OFF)

Enable or disable the "fuzzy timer".  The timer is off by

default, because in the normal case KERMIT-80 is communicating

with a mainframe KERMIT that has its own timer.  Mainframe

KERMIT timers tend to be more precise or adaptable to changing

conditions.  You should SET TIMER ON if you are communicating

with a KERMIT that does not have a timer.  You should SET TIMER

OFF if you are communicating over a network with long delays.

USER user-number

Sets another user number to be active.  Acceptable user numbers

are 0 to 31, though it is recommended to use user numbers 0 to

15 only.  This is really only useful for Winchester Systems

with high disk capacities.

WARNING ON (or OFF)

Warn user of filename conflicts when receiving files from

remote host, and attempt to generate a unique name by adding

"&" characters to the given name.  ON by default, which is

equivalent to SET COLLISION RENAME.

SHOW    Display all settable parameters.  You will get a page or so of the

status af all parameters that can be set using the SET command.

STATUS  The same function as Show.

STAY    Equivalent to SET NO-EXIT.

TAKE filespec

Take characters and commands from the specified file as if they were

entered from the keyboard.  This is useful if you want to set up a

batch job.  A command file can send, get, receive, set functions etc

automatically.  A TAKE command can be interrupted with ^C.

An automatic "TAKE KERMIT.INI" is executed from the default drive when

Kermit-80 is loaded.  This can be used to set defaults of band rate,

parity, filetype, default drive etc.

If KERMIT.INI does not exist, control is given directly to the user.

TRANSMIT filespec turnaround

Send the specified file to the system on the other end of the

connection as though it were being typed at the terminal, one line at a

time.  Each line sent is terminated with a carriage return, and any

line feeds are stripped from the file sent.  After each line has been

sent Kermit waits for a character string from the host (eg a acrriage

return).  If not specified, a carriage return is assumed.  No KERMIT

protocol is involved.  An asterisk (star) is sent to the console for

every line sent, to indicate how the transfer is progressing.  This is

useful for sending files to systems that don't have a KERMIT program.

During transmission, you may type one of these single-character

commands:

Control-C

Cease transmission, and drop into terminal emulation mode.

CR  (carriage return) Re-transmit the previous line.

TYPE filespec

Type a local CP/M file or files on the CP/M screen.  A Control-C will

cancel the command and return to the Kermit prompt.  A Ctrl-X will

cancel the current file and go on to the next one, if any.  Typing any

other character while the file is being displayed will suspend the

output.  Another character will resume output.

VERSION Show the name, edit number, and edit date of several of the modules

that make up Kermit-80.

1.6. Kermit-80 Flavors

Many of the systems supported use an external terminal, rather than a built-in

console.  Kermit may be further customized for these systems by defining (at

assembly time) the terminal type to be used.  If the terminal type is unknown

or does not match any of the existing terminal options, the generic "CRT"

option may be selected.  In this case, Kermit cannot do fancy screen control

during file transfer; it simply types the file names, packet numbers, and

messages in sequence across and down the screen.  This works best if you can

put your micro or terminal in "autowrap" mode; otherwise the packet numbers

will pile up in the rightmost column; the filenames and messages will always

appear on a new line, however.  If no specific terminal has been selected,

Kermit cannot do VT52 emulation; it can act as a "dumb terminal" (sometimes

called a "glass TTY"), or else its own built in terminal firmware provides

cursor control functions independent of the Kermit program.

1.6.1. Generic Kermit-80

"Generic Kermit-80" is an implementation of Kermit that should run on any 8080-

compatible CP/M 2.2 system with no modification at all, or perhaps only a minor

one.  Unlike other Kermit-80 implementations, it contains no system-dependent

manipulation of the serial port.  All I/O is done with standard CP/M BIOS

calls, and I/O redirection is done using the CP/M IOBYTE function, which,

according to the Digital Research CP/M Operating System Manual, is an optional

feature of any particular CP/M implementation.  If your system does not provide

the IOBYTE function, Generic Kermit-80 will not work; furthermore, not all

systems that implement IOBYTE do so in the same way.  The SET PORT command may

be used to select the devices to be used for input and output.  Table 1-1 lists

the options to the SET PORT command and their effects.

-----------------------------------------------------------------------

SET PORT xxx    input from      output to

CRT            CRT:            CRT:

PTR            PTR:            PTP:

TTY            TTY:            TTY:

UC1            UC1:            UC1:

UR1            UR1:            UP1:

UR2            UR2:            UP2:

Table 1-1:  Kermit-80 SET PORT Options

-----------------------------------------------------------------------

The default is SET PORT PTR.  In all cases, the console (CON:) and list (LST:)

devices used are those selected when Kermit is started.

The reason all Kermit-80 implementations aren't generic is that a good deal of

speed is sacrificed by getting all services from the operating system.  While a

specific implementation of Kermit-80 may be able to operate at 4800, 9600, or

even 56 Kilo baud, generic Kermit will fail to work on some systems at speeds

in excess of 1200 baud.  In addition, many features of Kermit require more

specific knowledge of the hardware involved.  Generic Kermit cannot send a

BREAK signal, or change the baud rate, for example.

1.6.2. CP/M 3 Kermit

CP/M-3 Kermit (also known as CP/M-Plus Kermit) is a version of generic

Kermit-80, and should run on most CP/M-3 (CP/M-Plus) systems.  It uses the

auxilliary port (AUX:) to communicate to the remote Kermit.  The SET BAUD and

SET PORT commands are not supported; nor can a BREAK be sent.  Like generic

Kermit-80, a terminal may be selected at assembly time.

1.6.3. System-Specific Versions

There are also many versions of Kermit-80 tailored to specific systems.  Most

of these operate uniformly, but some of them take advantage (or suffer

limitations) of the specific system.  Here are some of the special features for

particular systems:

Amstrad: -- Two versions:

PCW 8256

The PCW 8256/8512 with the serial inerafce attached.

CPC 6128

The 664 with add on memory and 6128 are both supported.  Both systems

must run CP/M Plus, so the 664 will need an add on RAM pack and CP/M

upgrade.  A high speed transfer rate of 38k baud can be used between

Amstrad computers.

ACCESS:

Access Matrix computer using port J5.  Supports SET BAUD-RATE for rates of

300-9600 baud.

Apple II -- four variations:

APMMDM:

Apple with Z80 Softcard and Micromodem II in slot 2 Dialout capability

provided in connect command; user is prompted for phone number if

carrier is not present.  During connect mode, ^]D drops carrier.  BYE

command also causes carrier to be dropped.

AP6551:

Apple with Z80 Softcard, and one of several 6551-based communication

cards; the slot number is a compile-time parameter (default is slot 2).

SET BAUD-RATE supported; speeds are 110-19200 baud.

APCPS:

Apple with Z80 Softcard and CP Multi-Function Card.  The slot number is

again a compile-time parameter.  SET BAUD-RATE is supported for baud

rates from 50 baud to 19200 baud.

AP6850:

Apple II with Z80 Softcard and a 6850-based USART in slot 2-the slot

being a compile-time parameter.  SET BAUD-RATE is not supported.

BBC:

Acorn Computers BBC Computer with Acorn Z80 second processor running

CP/M-80.  Supports SET BAUD-RATE and can send breaks.

BigBoard II:

Uses serial port A. To use port B, change mnport, mnprts, and baudrt and

reassemble.  Can generate BREAK.  SET SPEED supported; speeds are 300-38400

baud.

Cifer:

Originally coded for Cifer 1886 using the VL: port set as TTYI: and TTYO:

but works successfully on 18xx and 28xx series machines.

There are now two versions, each with two variations: Either running CP/M

Version 2.2 or 3.0, and either using the VL: or AUX: ports.  The VL:  port

version can only use seven bits of data, so parity prefixing is required

for binary file transfers.  This restriction is removed by using the AUX:

port.  For those interested, the problem is due to the interprocessor link

between the video and CPU (!) boards.  The VL: port is on the video board,

and the AUX: port on the CPU board, and the inter processor link can only

transfer seven bits of data.

Supports SET SPEED, and can generate breaks on some models with a BREAK

key.

Comart:

Comart Communicator-Similar to Northstar equipment.  Can generate BREAK.

Compupro:

Based on Kermit 3.x, and has been merged into V4.09

CPT-85xx word processors:

Can generate BREAK.  SET SPEED supported; speeds are 50-9600 baud.

Cromemco:

Cromemco computers with TU-ART card.  Supports SET BAUD-RATE (110-9600

baud).

DEC DECmate II word processor (with Z80 card):

Can generate BREAK.

DEC VT180 (Robin):

Three output ports, referred to as COMMUNICATIONS, GENERAL, and PRINTER.

Can generate BREAK.

Digicomp Delphi 100:

SET SPEED supported; speeds are 50-19200 baud.

Discovery:

Action Computer Enterprises "Discovery" Multi-user Computer.  Uses Port B

on an 83U user board.  Supports SET SPEED for 50-19200 baud.  Can generate

BREAK.

Epson:

Epson PX-8 with LCD display.  Although it is quite different in displaying

of Packet Information, it works as any other CP/M-80 Kermit.  Supports SET

SPEED and can generate BREAK.

Generic Kermit:

Two versions, one for CP/M version 2.2 and version 3.  These systems use

IOBYTE flipping (V2.2) and the AUX:  device to communicate to the serial

line.  You may have to SET PORT xxx before the version 2.2 will work, as

Kermit needs to know what device is the serial line.

Genie:

Eaca Video Genie.

Heath: Three Versions:

H8QUAD

for Heath-8 systems with the quad io board.  This system has been

derived from V3.x code.  Note that this version will not run "as is" on

H89 systems.

H89 For Heath-89 machines suports baud rates from 50 to 56,000 baud.

Z100

For Z-100 running CP/M-85.  This version does not support setting of

baud rates.

Intertec Superbrain: Two Versions:

BRAINA

For superbrain using AUX port.  Breaks and SET BAUD both supported

BRAINM

As above, but using the MAIN port.

Ithaca:

Ithaca Intersystems based computer using the VIO card for all IO to the

outside world.  The system is strictly speaking a home-brew variant of the

Ithaca machine, using an S100 cardcage without a front panel.  It uses the

Extended BIOS by EuroMicro of London.  However, I see no reason for this

version not running on a genuine Ithaca Intersystems machine.  There are

patches needed to the EuroMicro BIOS to make this version work.

Kaypro:

Should work on most Kaypro models, as well as some related systems

(Ferguson BigBoard I, Xerox 820).  For the newer Kaypros with multiple

ports, Kermit uses the one labeled "serial data"; it cannot use the serial

printer or internal modem ports (but it should be possible to modify the

values for mnport, mnprts, and baudrt to do this).  Can generate BREAK.

SET SPEED supported; speeds are 50-19200 baud.

Lobo:

Lobo MAX-80.  Supports SET SPEED and can generate BREAKS.

Merlin:

British Telecom Merlin M2215 (also Rair Black Box, possibly also the ICL

PC?).  Requires a terminal.

Microbee:

Microbee Systems computer made in Australia.  Works on Microbee 56K (Series

2 APC), 64K (Computer in a Book), 128K (Dynamic), and 256K (256TC).  Can

generate BREAK.  SET BAUD-RATE supported; speeds are 75-9600.  All serial

I/O is via software, not hardware.  Simultaneous transmit and receive

possible on all speeds except 75/1200, 1200/75, 4800, and 9600.

Micromate:

PMC 101 Micromate.  Supports SET SPEED and can generate BREAK.

Micromint: Two versions

S6  The Ciarcia/Micromint sb-180 board with a 6Mhz procoessor.  System

requires a terminal.

S9  As above, but with a 9Mhz processor.

NCR:

Decisionmate 5.  Uses the 2651 and is largely the same as the Ithaca

Intersystems machine implementation.

Northstar: -- There are four versions available:

NORTHS:

Northstar Horizon with HS10-4 board.  Supports SET SPEED and SET PORT.

HORIZON:

Northstar Horizon using the serial ports on the Mother board.  Can

generate BREAK.

BASICNS:

Basic Northstar Horizon using the printer port.  Can generate BREAK.

ADVANT:

Northstar Advantage.  Supports SET SPEED and can generate BREAK.  Traps

Control-0 in the system filter.

Morrow Decision I:

Uses the Multi-I/O board.  Port 1 is the console, port 3 is the

communications line.  SET SPEED supported; speeds are 75-56000 baud.

Morrow Micro Decision I:

Nokia MicroMikko:

Will not echo control-O (which locks keyboard).  SET SPEED supported;

speeds are 75-9600 baud.

Ohio Scientific:

Doesn't have screen control.

Osborne 1:

Uses serial line, not internal modem.  Left-arrow key generates <DEL>

("delete" or "rubout" character) during connect mode.  SET SPEED supported;

speeds are 300 and 1200 baud.  Now supports multi-sector buffering.

Research Machines: Two Versions:

RM380ZM:

380Z and 5.25" disks supports SET BAUD.RATE

RM380ZF:

380Z and 8" disks, otherwise as above.

Sanyo:

Sanyo MBC-1100.  This version derived from Kermit V3.x

ScreenTyper:

Details unkown.

TRS-80: Three versions:

TRS80LB:

TRS-80 with Lifeboat CP/M

TRS80PT:

TRS-80 with Pickles and Trout CP/M

TRSM4:

TRS-80 Model 4 with Montezuma CP/M

Teletek:

Teletek Systemaster. Supports SET BAUD.

Telcon:

TELCON ZOBRA portable computer.

Torch:

Torch Unicorn 5 initially, but the code runs on other Z80 based CP/N (as in

Nut!) systems.  It uses the BBC Computer as a "Base processor", and is

similar to the BBC version.  The base processors RS423 port is used rather

than any built in Modem.  (UK telecoms legislation effectively makes modem

control software tricky business...).  Two potential versions exist-one

using cursor positioning codes for a MCP and CCCP ROM combination of

revision less than 1.00, the other version uses the additional facility

MCP/CCCP versions greater than 1.  Supports SET SPEED and can generate

BREAKs.

Note that binary files must be transferred using SET PARITY to anything

other than NONE!  Parity is neither generated nor checked.

US Micro Sales:

S-100-8 based computer.

Vector Graphics:

Vector

Xerox:

Xerox 820.

Z80MU:

Development Kermit on a PC running the Z80MU Z80 and CP/M 2.2 development

system.  Allows development of the system independent modules to be done on

an IBM PC or clone.  Also allows the generation of new .HEX files, that may

then be KERMITed to the target system.  Note:  Not all the BDOS or BIOS

routines are supported, so avoid "unusual" BIOS/BDOS calls.  (For example,

DIR from within Kermit will fail as one of the BIOS routines returning disk

parameters is not supported.)

1.7. Installation of Kermit-80

Kermit-80 was written originally for the Intertec SuperBrain in lowest-common-

denominator  8080 code with the standard assembler, ASM (single source module,

no macros, no advanced instructions), so that it could be assembled on any

CP/M-80 system (the 8080 assembler is distributed as a standard part of

CP/M-80, whereas the fancier Z80 or macro assemblers are normally commercial

products).  It has since been modified to run on many other systems as well.

Kermit-80 should be able to run on any 8080-, 8085- or Z80-based microcomputer

under  CP/M with appropriate minor changes to reflect the port I/O and screen

control for the system (see below).

The proliferation of new systems supported by Kermit-80 made the program grow

so large and complicated that it had to be broken up into system-independent

and system-dependent modules, as of version 4 (this was done by Charles

Carvalho of ACC).  Each module is composed of multiple files.  This has reduced

the time and disk space necessary for assembly; Kermit-80 may once again be

assembled on a CP/M system with roughly 250Kbytes of space.  The majority of

the code does not need to be reassembled to support a new system.

Unfortunately, it can no longer be assembled with ASM, since ASM does not

support multiple input files.  To allow it to be assembled on any CP/M system,

the public-domain assembler LASM is included in the distribution kit. Kermit-80

may also be assembled with Microsoft's M80 (not supplied).  In theory, any 8080

assembler supporting the INCLUDE directive ought to work, as well.

All versions of Kermit-80 are assembled from the same set of sources, with

system dependencies taken care of by assembly-time conditionals within the

system-dependent module (eventually, the system-dependent module will itself be

broken up into multiple files, one for each system).  The most important system

dependencies are terminal emulation (when CONNECTed to the remote host) and

screen handling, which are dependent on the individual micro's escape codes

(these features are table driven and easily modified for other CP/M systems),

and the lowest level I/O routines for the serial communications port.  The port

routines are best done only with BDOS calls, but some systems do not allow

this, primarily because the BDOS routines strip the parity bit during port I/O,

and the parity bit is used for data when transmitting binary files.

Kermit-80's I/O routines must check the port status and go elsewhere if no

input is available; this allows for virtual terminal connection, keyboard

interruption of stuck transmissions, etc.  On systems that fully implement I/O

redirection via the optional CP/M IOBYTE facility, this may be done by

switching the IOBYTE definition.  On others, however, IN/OUT instructions

explicitly referencing the port device registers must be used.

CP/M-80 KERMIT versions 3.8 and later include a "fuzzy timer" that allows a

timeout to occur after an interval ranging from 5 to 20 seconds (depending upon

the speed of the processor and the operating system routines) during which

expected input does not appear at the port.  In this case, retransmission

occurs automatically.  In any case, you may type a carriage return during

transmission to simulate a timeout when the transfer appears to be stuck.

1.7.1. Organization of Kermit-80

Kermit-80 consists of two modules, each of which is generated from multiple

source files.  The first module contains the system-independent code; the

second module is configured for a particular system and merged with the system-

independent module to produce a customized Kermit-80.

The distribution kit contains:

- the system-independent module, CPSKER.HEX;

- the system-dependent modules, CPV*.HEX (see table 1-2 and 1-3);

- the source files, CPS*.ASM and CPX*.ASM,

- the public-domain CP/M assembler, LASM.*,

- the public-domain CP/M load/patch utility, MLOAD.*

-------------------------------------------------------------------------------

Symbol  Filename System

ACCESS  CPVACC  Access Matrix

ADVANT  CPVADV  Northstar Advantage

AP6551  CPVAPL  Apple II, Z80 Softcard, 6551 ACIA in serial interface

AP6850  CPVA65  Apple II, Z80 Softcard, 6850 ACIA in Serial Iiterface

APMMDM  CPVAPM  Apple II, Z80 Softcard, Micromodem II in slot 2

APCPS   CPVCPS  Apple II, Z80 Softcard, with CPS multifunction card

BASICNS CPVBNS  Northstar Horizon (terminal required)

BBC     CPVBBC  Acorn "BBC" computer with Acorn Z80 second processor

BBII    CPVBB2  BigBoard II (terminal required)

BRAINM  CPVBRM  Intertec Superbrain using the main port

BRAINA  CPVBRA  Intertec Superbrain using the Aux port

CIFER2  CPVCIF  Cifer 1886 using the VL: Serial port and CP/M V2.2

CIFER3  CPVCI3  Cifer 1886 using the VL: Serial port and CP/M V3.0

CIFER2  CPVCA2  Cifer 1886 using the AUX: Serial port and CP/M V2.2

CIFER3  CPVCA3  Cifer 1886 using the AUX: Serial port and CP/M V3.0

CMEMCO  CPVCRO  Cromemco with TU-ART card. Terminal required)

COMART  CPVCOM  Comart Communicator (terminal required)

COMPRO  CPVPRO  Compupro with Interfacer 4 (or 3).  Terminal required.

CPC     CPVCPC  Amstrad CPC 664 and 6128 and CP/M 3

CPM3    CPVCP3  "Generic": CP/M 3.0 (CP/M Plus) systems (terminal req'd)

CPT85XX CPVCPT  CPT-85xx wordprocessor with CP/M

DELPHI  CPVDEL  Digicomp Delphi 100 (terminal required)

DISC    CPVDIS  Action Computer Enterprises "Discovery" (terminal req'd)

DMII    CPVDM2  DECmate II with CP/M option

GENER   CPVGEN  "Generic": CPM 2.2 systems with IOBYTE (terminal req'd)

GENIE   CPVGNI  Video Genie

H8QUAD  CPVH8Q  Heath-8 with Quad 8 i/o board

HEATH   CPVH89  Heath/Zenith H89

HORIZON CPVHOR  Northstar Horizon (terminal required)

KPII    CPVKPR  Kaypro-II (and 4; probably supports all Kaypro systems)

LOBO    CPVLBO  Lobo Max-80

"symbol" is the symbol used to select the target system, in CPVTYP.ASM;

"filename" is the name under which the module is supplied in the distribution.

Table 1-2:  Systems supported by Kermit-80 (Part 1)

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Symbol  Filename System

M2215   CPVMRL  British Telecom Merlin/Rair Black Box (terminal required)

MBEE    CPVBEE  Microbee

MDI     CPVMDI  Morrow Decision I (terminal required)

MIKKO   CPVMIK  MikroMikko

MMATE   CPVMM   PMC 101 Micromate (terminal required)

MMDI    CPVUD   Morrow Micro Decision I (terminal required)

NCRDMV  CPVDMV  NCR Decision Mate V.  (Terminal required?)

NORTHS  CPVNS   Northstar Horizon with HSIO-4 card (terminal req'd)

OSBRN1  CPVOSB  Osborne 1

OSI     CPVOSI  Ohio Scientific

PCI2651 CPVPCI  Ithaca Intersystems with VI0 card (terminal required)

PCW     CPVPCW  Amstrad PCW 8256/8512 with serial interface

PX8     CPVPX8  Epson PX-8

RM380ZM CPVRMM  Research Machines 380Z with MDS (5.25" discs)

RM380ZF CPVRMF  Research Machines 380Z with FDS (8" discs)

ROBIN   CPVROB  DEC VT180

S1008   CPVUSM  US Microsales S-100-8 (terminal required)

SANYO   CPVSAN  Sanyo MBC-1100

SB6     CPVSB6  Micromint SB-180 with 6Mhz CPU (terminal required)

SB9     CPVSB9  Micromint SB-180 with 9Mhz CPU (terminal required)

SCNTPR  CPVSCN  Screentyper

TELCON  CPVTEL  TELCON Zobra portable

TELETEK CPVTET  Teletek Systemaster

TORCH   CPVTRC  Torch computers BBC-B with Z80 second processors

TRS80LB CPVTLB  TRS-80 model II with Lifeboat 2.25C CP/M Display

TRS80PT CPVTPT  TRS-80 model II with Pickles + Trout CP/M Display

TRSM4   CPVTM4  TRS-80 model IV

VECTOR  CPVVEC  Vector Graphics

XER820  CPVXER  Xerox 820

Z100    CPVZ00  Z-100 under CP/M-85

Z80MU   CPVZ80  Z80MU development system on a PC

"symbol" is the symbol used to select the target system, in CPXTYP.ASM;

"filename" is the name under which the module is supplied in the distribution.

Table 1-3:  Systems supported by Kermit-80 (Part 2)

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Symbol  Terminal type

CRT     Dumb terminal type.  Does not do cursor addressing

ADM3A   Lear Seigler ADM 3A

ADM22   Lear Seigler ADM 22

AM230   Ampro 230

H1500   Hazeltine 1500

SMRTVD  Netronics Smartvid

SOROQ   Soroq IQ-120

TVI912  Televideo 912

TVI925  Televideo 925 or Freedom 100

VT52    Dec VT52 or equivalent (H19)

VT100   Dec VT100 or equivalent

WYSE    Wyse 100

"symbol" is the symbol used to select the target system, in CPXTYP.ASM;

"Terminal type" is the type of terminal "symbol" selects.

Table 1-4:  Terminals supported by Kermit-80

-------------------------------------------------------------------------------

1.7.2. Downloading Kermit-80

You'll need either a pre-configured .COM file or the system-independent module,

CPSKER, in binary (.COM) or hex (.HEX) format and the system-dependent overlay

for your system (from Tables 1-2 and 1-3).  If your system is not listed in the

table, get the generic CP/M 2.2 Kermit or the generic CP/M 3 Kermit.  If you

already have a version of Kermit on your micro and you want to install a new

version, simply use your present version to get the new files.  Transfer the

files to your system and skip ahead to "merging the modules".

If you do not have a copy of Kermit on your micro, and you cannot borrow a

Kermit floppy but you do have access to a mainframe computer with a copy of the

Kermit-80 distribution, you should read this section.

There are several ways to get CP/M Kermit from a host system to your micro.

The easiest is to "download" the necessary "hex" files into your micro's memory

and then save them on the disk.  If you have a terminal emulator program on

your micro which can save a copy of the session to disk, connect to your host,

and type the necessary files.  Exit from the emulator, saving the session log,

and edit the session log to extract the hex files.  Skip ahead to "merging the

files".

The following is a procedure which, though far from foolproof, should allow you

to get a version of Kermit to your CP/M based micro.  It depends upon the host

prompt, or at least the first character of the host prompt, being some

character that cannot appear in a hex file (the valid characters for hex files

are the digits 0-9, the upper case letters A-F, the colon ``:'', carriage

return, and line feed).  As soon the prompt character is encountered, the

transfer will terminate.  If your host does not issue a prompt that will

accommodate this scheme, you can achieve the same effect by adding an atsign

``@'' to the very end of the hex file before sending it from the host.  The

program below looks for an atsign (the normal DEC-20 prompt, hex 40).

DECSYSTEM-10 users would look for a dot, hex 2E; VAX/VMS or UNIX users would

look for a dollar sign, hex 24; UNIX C-Shell users would look for a percent

sign, hex 26.

1. For CP/M 2.2 systems, connect to a floppy disk with plenty of free

space.  Run DDT and type in the following (the comments should not

be typed in; they are there just to tell you what's happening):

(Note that this wont work for CP/M Plus or 3.0 systems!)

----------------------------------------------------------------

-a100                   ;Begin assembling code at 100

0100    LXI H,2FE       ;Where to store in memory

0103    SHLD 200        ;Keep pointer there

0106    MVI E,D         ;Get a CR

0108    MVI C,4         ;Output to PUNCH (send to HOST)

010A    CALL 5

010D    MVI C,3         ;Input from READER (read from HOST)

010F    CALL 5

0112    ANI 7F          ;Strip parity bit

0114    PUSH PSW        ;Save a and flags

0115    MOV E,A         ;Move char to E for echo

0116    MVI C,2         ;Output to screen

0118    CALL 5

011B    POP PSW         ;Restore A and flags

011C    CPI 40          ;(or 4E,24,26,etc) System prompt?

011E    JZ 127          ;Yes, have whole file in memory

0121    CALL 17A        ;No, store another byte

0124    JMP 10D         ;Read another byte

0127    MVI A,1A        ;Get a Control-Z (CP/M EOF mark)

0129    CALL 17A        ;Store it in memory

012C    LXI H,300       ;Get memory pointer

012F    SHLD 202        ;Store as DMA pointer

0132    LDA 201         ;Get 'HI' byte of memory pointer

0135    STA 200         ;and store it as 'LO' one

0138    XRA A

0139    STA 201         ;Zero 'HI' byte (slow *256)

013C    MVI C,16        ;Make NEW file

013E    LXI D,5C        ;With FCB1

0141    CALL 5

0144    CALL 15E        ;Write 128 bytes (sector)

0147    CALL 15E        ;Write another sector

014A    LXI H,FFFF      ;Get a 16-bit Minus One

014D    XCHG            ;into DE

014E    LHLD 200        ;Get 256-byte counter

0151    DAD D           ;Decrement

0152    SHLD 200        ;and store back

0155    MVI A,2         ;Check if

0157    CMP L           ; 256-byte counter down to offset

0158    JZ 183          ;Yes, we're done

015B    JMP 144         ;Keep writing..

015E    LHLD 202        ;Get file-pointer

0161    XCHG            ;into DE

0162    MVI C,1A        ;Set DMA-address

0164    CALL 5

0167    MVI C,15        ;Write sector (128 bytes)

0169    LXI D,5C        ;using FCB1

016C    CALL 5

016F    LHLD 202        ;Get file-pointer

0172    LXI D,80        ;128-bytes

0175    DAD D           ;added to file-pointer

0176    SHLD 202        ;and save

0179    RET             ;and return

017A    LHLD 200        ;Get Memory-pointer

017D    MOV M,A         ;Store character

017E    INX H           ;Increment Pointer

017F    SHLD 200        ;and save

0182    RET             ;and return

0183    MVI C,10        ;CLOSE file

0185    LXI D,5C        ;using FCB1

0188    CALL 5

018B    JMP 0           ;Force WARM BOOT

0179

-^C                     ;(Type Control-C) Return to CP/M

A>SAVE 1 FETCH.COM      ;Save program, we need to run it twice.

Figure 1-1:  Bootstrap program for Kermit-80 and CP/M Version 2.2

----------------------------------------------------------------

Alternatively, an assembler source file for this program is

distributed with CP/M Kermit as CPKFET.ASM.  You might prefer to

type the assembler version in and assemble and load it (ASM CPKFET,

LOAD CPKFET, or MASM CPKFET, MLOAD CPKFET), to let the assembler and

loader catch any typing errors.

2. Connect to your host using a terminal or a terminal emulation

program.  Ensure that your host does not have your terminal in "page

mode" (does not pause at the end of each screenful).

3. Tell the host to display the first hex file (the system-independent

module) at your terminal, e.g. give a command like TYPE CPSKER.HEX,

without a terminating carriage return.

4. Return to your micro by switching the cable from the terminal to the

micro, or by terminating the micro's terminal program.

5. Make sure your IOBYTE is set so that RDR: and PUN: correspond to the

I/O port that is connected to the host (this would normally be the

case unless you have done something special to change things).

6. Load the program you entered in the first step with DDT, and use it

to capture the first hex file:

DDT FETCH.COM

-icpsker.hex            ;Setup FCB for file CPSKER.HEX

-g100,179               ;Execute the program.

Now there should be a file CPSKER.HEX on your connected disk.

7. Return to the host, and tell it to display the second hex file (the

system-dependent module for your configuration).  Again, do not type

the terminating carriage return.

8. Return to your micro, and run the capture program again:

DDT FETCH.COM

-icpxovl.hex            ;Setup FCB to create CPXOVL.HEX

-g100,179               ;Execute the program.

Now there should be a file CPXOVL.HEX on your connected disk.

Replace CPXOVL.HEX in this example with the appropriate overlay file

for your system.

Merging the files:

1. For purposes of illustration, we will assume the system-dependent

overlay is called "cpxovl.hex".  The two hex files may be combined

with MLOAD or DDT.  If you already have a running Kermit, you can

transfer MLOAD.HEX to your system and create MLOAD.COM by running

LOAD.  If you're bootstrapping Kermit, you could transfer MLOAD.HEX

to your system the same way you got the other two .HEX files, but

it's probably simpler to use DDT to get Kermit running, and get

MLOAD later if you need it.

2. Using MLOAD, the two pieces may be easily merged:

A>mload kerm411=cpsker,cpxovl

(Some messages about program size, etc...)

A>

3. If you don't have MLOAD running, it's a bit more complex:

A>ddt cpsker.hex

NEXT  PC

3500 0100

-icpxovl.hex

-r

NEXT  PC

xxxx 0000

-^C

A>save dd kerm411.com

The page count ("dd") used in the SAVE command is calculated from

the last address ("xxxx") given by DDT in response to the R command:

drop the last two digits and add 1 if they were not zero, then

convert from hexadecimal (base 16) to decimal (base 10): 684F

becomes 69 hex, which is 105 decimal (5 times 16 plus 9) -- but 6700

becomes 67 hex, or 103 decimal (consult an introductory computing

book if you don't understand number base conversion).

4. If you are using the Z80MU CP/M and Z80 development toolkit on an

IBM PC or clone, then follow the same instructions as for a genuine

CP/M system.  When you have loaded your file, you will have to ship

the .COM or two .HEX files to the target CP/M system. (Possibly

using a previous issue of Kermit?)

5. Note that CP/M hex files have checksums on each line.  If there were

any transmission errors during the downloading process, MLOAD or DDT

will notice a bad checksum and will report an error (something like

"Illegal Format").  If you get any errors during loading, either fix

the hex file locally with an editor, or repeat the transfer.

You now should have a running version of Kermit-80, called KERM411.COM.

Test your new Kermit by running it.  If it gives you a prompt, it might be OK.

(don't delete your old one yet...).  Instead of a prompt, you could get one of

two messages indicating that the configuration information is invalid:

?Kermit has not been configured for a target system

or

?Consistency check on configuration failed

Of course, neither of these messages should appear if you're building Kermit

from the distribution kit.  The first message indicates that the overlay was

not found where the system-independent module expected to find it, probably

because the overlay address is incorrect; the second indicates that the version

of CPXLNK used in the system-dependent module is incompatible with the system-

independent module.

Once you are satisfied that KERMIT40 works correctly, you should rename your

old KERMIT.COM to something else, like OKERMIT.COM, and rename KERMIT40.COM to

KERMIT.COM.

1.7.3. Assembling Kermit-80 from the sources

Kermit-80 is built in two pieces from the following files:

The system-independent files:

CPSKER.ASM    header file

CPSDEF.ASM    definitions for both KERMIT and KERSYS

CPSMIT.ASM    initialization, main loop, miscellaneous commands (BYE, EXIT,

LOG, SET, SHOW, STATUS, and VERSION)

CPSCOM.ASM    second part of commands, status and set file

CPSPK1.ASM    part 1 of the KERMIT protocol handler (SEND, RECEIVE,

LOGOUT,and FINISH commands)

CPSPK2.ASM    part 2 of the KERMIT protocol handler

CPSREM.ASM    REMOTE routines (FINISH, BYE and LOGOUT in CPXPK*.ASM)

CPSSER.ASM    SERVER routines (for the future)

CPSTT.ASM     the transparent commands (TRANSMIT, CONNECT)

CPSCPM.ASM    CP/M commands (DIR, ERA, USER, TYPE, PRINT, COPY)

CPSWLD.ASM    the wildcard handler

CPSCMD.ASM    the command parser

CPSUTL.ASM    utility routines and data

CPSDAT.ASM    data space and the overlay definitions

CPXLNK.ASM    linkage area description

The system-dependent files:

CPXTYP.ASM    system selection

CPXLNK.ASM    system overlay specification and jump table

CPXCOM.ASM    common routines for all systems

CPXSWT.ASM    system selector or switcher

One of:

CPXSYS.ASM    family file for some system-specific code

CPXTOR.ASM    family file for Torch, Superbrain, PCI2651 etc

CPXNOR.ASM    family file for Northstar and Comart machines

CPXMRL.ASM    family file for British Telecom Merlin/Rair Black Box

CPXSB.ASM     family file for Micromint SB-180 systems

CPXCIF.ASM    family file for Cifer systems

CPXHEA.ASM    family file for Heath/Zenith systems

CPXAPP.ASM    family file for Apple II systems

CPXPCW.ASM    family file for Amstrad PCW 8256/8512 machines

CPXBBI.ASM    family file for BigBoard, Kaypro and Xerox 820 systems

CPXBEE.ASM    Microbee

CPXSYO.ASM    family file for Sanyo MBS-1100 systems

CPXTM4.ASM    family file for Tandy Model 4 with CP/M systems

CPXGNI.ASM    family file for Video Genie systems

CPXPRO.ASM    family file for Compupro systems

CPXZ80.ASM    family file for the Z80MU development system

and if you use a terminal,

CPXVDU.ASM    display codes for VDUs etc.  Not always required

The system-independent module contains all of the system-independent files

except for CPXLNK.ASM, which is assembled into the system-dependent module to

provide the structures needed to connect the two modules.  As distributed, the

system-independent module is named CPSKER.HEX.  If you have a copy of

CPSKER.HEX, you do not need to reassemble the system-independent module to

configure Kermit for your system.

The system-dependent module consists of CPXTYP.ASM, CPSDEF.ASM, CPXLNK.ASM,

CPXSWT.ASM, CPSCOM.ASM, one of the family files CPXSYS.ASM, CPXTOR.ASM,

CPXMRL.ASM, CPXSB.ASM, CPXCIF.ASM, CPXHEA.ASM, CPXBBI.ASM, CPXTM4.ASM,

CPXGNI.ASM, CPXNOR.ASM, CPXAPP.ASM, CPXPCW.ASM, or CPXPRO.ASM, and possibly

CPXVDU.ASM, if your system uses a terminal for the console.  One copy of the

system-dependent module is supplied already assembled for each supported

system; the filename may be obtained from tables 1-2 and 1-3.  If a terminal is

required for a system, a CRT (glass TTY device) has been selected.

After assembling the two pieces separately, they are combined with DDT or MLOAD

into a system-specific Kermit.

If you want to rebuild the system-independent module, the only change you may

need to make is to select the assembler to be used, in CPSKER.ASM.  Define one

of MAC80, M80, or LASM to TRUE to select it as the assembler; the others should

be defined FALSE.

Assuming you have the Microsoft Macro Assembler package (M80/L80), you'll need

to do the following:

A>m80 cpsker=cpsker.asm

A>l80 /p:100,cpsker,cpsker/n/e

This will produce CPSKER.COM.

If you are using LASM instead, do this:

A>lasm cpsker

LASM will generate CPSKER.HEX and CPSKER.PRN.  LASM allows options to be

specified in the same way as the standard assembler, ASM, so the command

A>lasm cpsker.abz

will read the source files from drive A, send the .HEX file to drive B, and

suppress the listing file.

If you are using the Z80MU development system on an IBM PC or clone, then

assemble your files using either LASM and MLOAD or M80 and L80, as if you were

using a genuine CP/M-80 system.  Note that you will still have the problem of

transferring your assembled files to the target CP/M system.

If you want to generate a system-dependent overlay for a particular system, or

want to change the terminal supported, you'll need to check three areas in

CPXTYP.ASM:

First, the overlay start ADDRESS.  The symbol "ovladr" is EQUated to the

address of "LNKFLG" in the system-independent module, as the starting address

of the overlay (7000H for version 4.11).  You'll need to know this value if

you're building the overlay with M80/L80.  You won't normally need to change

this value.

Second, the assembler being used.  Again, define one of MAC80, M80, and LASM to

be TRUE to select it, and define the others to be FALSE.  The two modules

(system-independent and system-dependent) do not need to be built with the same

assembler.

Third, the system configuration.  Locate your system in tables 1-2 and 1-3,

then define the appropriate symbol TRUE, and the rest FALSE.  If the system

comes with a builtin console terminal, define all the terminal switches FALSE.

If the system uses an external terminal as the console, locate the terminal in

table 1-5 and define the appropriate symbol TRUE, and the remainder FALSE.  If

the terminal is not listed in table 1-5, use the CRT switch; in this case, VT52

emulation is not supported.

In addition, there are a few general and system-specific symbols which may be

altered to fit your system:

APSLOT          For Apple with 6551 ACIA, defines the slot number of the serial

card

CPUSPD          Processor speed in units of 100KHz (currently used only for

bbII and kpII for timing loops)

TAC             For users connecting through ARPAnet TACs: set to TRUE if you

wish the default TACTRAP status to be ON. (This may be

overridden with the SET TACTRAP command).  If you're not

connecting through a TAC, set tac to FALSE and ignore tacval.

TACVAL          For ARPANET TAC users: defines the default TAC intercept

character (may be overridden with the SET TACTRAP command).

If you are just assembling an existing configuration, you'll need to edit

CPXTYP.ASM only.  If you are adding support for a new system, you should not

modify CPSDEF.ASM or CPXLNK.ASM; if you do, you'll have to change the system-

independent module also.  Eventually, CPXSYS.ASM will be split into separate

files, each of which will generate one or more related systems.  When this

happens, you'll want to pick the one closest to your system to use as a

starting point.

After editing CPXTYP.ASM as necessary, assemble and link the overlay as

follows:

- With M80 (where "xxxx" is the hex value of ovladr from CPXLNK.ASM):

A>m80 cpxtyp=cpxtyp.asm

A>l80 /p:xxxx,cpxtyp,cpxtyp/n/x/e

- With LASM:

A>lasm cpxtyp

With an IBM PC or clone using the Z80MU softwrae, follow the instructions as if

you were using a real CP/M system.

The overlay (CPXTYP.HEX) may then be merged with the system-independent module

as described above (creating a runnable Kermit from the distribution kit).

If you are using the Z80MU development system on a PC, and already have a

running Kermit-80 v3.9 or later, you can merge the two .HEX files into a .COM

file with LINK80 (TOPS 10/20), MLOAD (Z80MU), L80 (Z80MU), and transfer the new

.COM file to your micro with Kermit:

- Z80MU on a PC and MLOAD:

@MLOAD KERNEW=CPSKER,CPXTYP

- Z80MU on a PC and C80:

@L80 /P:xxxx,CPXTYP,CPXTYP/N/X/E

producing KERNEW.COM.

-------------------------------------------------------------------------------

Symbol         Terminal description

crt            Basic CRT, no cursor positioning

adm3a          ADM3A Display or lookalike

adm22          ADM22 Display or lookalike

am230          Ampro 230

h1500          Hazeltine 1500

smrtvd         Netronics Smartvid-80

soroq          Soroq IQ-120

tvi912         TVI 912

tvi925         TVI 925, Freedom 100

vt52           VT 52 or VT52 emulator such as Heath H19, H29, etc.

vt100          VT 100 or emulator (most ANSI terminals should work)

wyse           Wyse 100

Table 1-5:  Terminals known to Kermit-80

-------------------------------------------------------------------------------

1.8. Adding Support For A New System

Kermit-80 is built from a common set of source files; the system-dependent

module makes heavy use of conditional assembly (this complication will be

removed in future releases).  The system dependencies arise from attempts to

answer some questions:

1. What kind of terminal is to be supported?

For many micros, the console is an integral part of the system, but

others can use an external terminal.  In either case, the commands

to manipulate the screen (position the cursor, erase the screen,

etc) must be defined.

2. How is the serial line accessed?

For systems supporting the IOBYTE function, this is straightforward;

the symbol "IOBYT" is defined TRUE.  If the serial line is accessed

with IN and OUT instructions, it may be possible to use the simple

I/O routines provided.  In this case, the symbol "INOUT" is defined

TRUE, the MNPORT and MNPRTS are defined to be the data and control

addresses, respectively, and bit masks for testing for "input data

available" and "output buffer empty" must be defined.  If the

interface is strange, leave IOBYT and INOUT set to FALSE, and

provide the I/O routines.

3. What initialization is necessary?

You may wish to set the baud rate or configure the serial line at

startup.  Examples for a number of devices are present.

4. What special features are to be supported?

You may want to provide the capability to select one of several

serial lines with the SET PORT command, or to change the speed of

the serial line with the SET SPEED command.  To do this, you'll need

to build a command table, using the systems already supported as

examples.  The ability to send a BREAK signal is desirable.  Again,

examples for several different interfaces (ACIA, SIO, etc) are

present.

5. Do you want to design an external terminal type?

There is a jump entry in the overlay file to allow users to add

their own termainl emulator.  If you write the code for such an

emulator, you must load this jump address with the address of your

emulator, and SET TERMINAL EXTERNAL from within Kermit.  All

characters will be passed to this routine during connect mode.

1.9. Notes on New Features in Kermit-80 Version 4

- Debugging aids:  SET DEBUG ON will add two fields to the SEND/RECEIVE

display, labelled "Spack" and "Rpack".  These display the last packet

sent and received.  Of course, this slows down the transfer,

especially if the console is an external terminal.  SET DEBUG OFF

removes these fields.  The VERSION command displays the name, edit

number, and edit date of several of the modules that make up Kermit.

- TAC support: ARPAnet TACs (and many other communication devices such

as terminal concentrators, modems, port contention units, network

PADs, etc) use a printing character (like "@") as an intercept

character, to allow commands to be issued to the TAC, or modem, etc.

In order to send this character to the host, it must be typed twice.

The command "SET TAC CHARACTER" to Kermit enables the "TACtrap" and

asks the user to specify the TAC intercept character.  This character

will be automatically doubled when it appears in Kermit protocol

messages (sent by the SEND or RECEIVE commands) or when it appears in

a file being sent with the TRANSMIT command.  It is not automatically

doubled when typed by the user in CONNECT mode.  "SET TAC ON" enables

the TACtrap but does not change the TAC intercept character, which is

initially "@".  "SET TAC OFF" disables the TACtrap.

- File buffering:  Previous versions of Kermit-80 buffered only one

sector (128 bytes) at a time during file transfer operations.  This

version buffers 16Kbytes at a time, reducing the number of times the

floppy drive must be spun up and down, and increasing the effective

throughput of the link.  If the disk transfer rate is too slow,

however, the remote Kermit may time out and retransmit packets.  This

will show up on the screen in the "Retries:" field; if this occurs

after disk activity, you may want to increase the timeout value on

the remote Kermit, SET BUFFER <new value> while in Kermit,or

reassemble Kermit with a smaller value for MAXSEC (in CPSDEF.ASM)

This buffer is also used by the TRANSMIT command; the log file

enabled by the LOG command is still written a sector at a time.

This section is intended for people wanting to implement their own versions of

Kermit-80 for computers not already defined.

The system independent code communicates to routines for a specific system

through a set of tables.  These tables are defined in CPXLNK.ASM, and should

not be modified between revisions of Kermit.  If an entry is added or deleted,

then the whole of Kermit-80 needs reassembling.  Make sure that the changes to

CPXLNK.ASM are duplicated in CPSUTL.ASM, which has the system independent

equivalent of CPXLNK.ASM.

The following entries/definitions apply to revision 4.09.  There have been

three additional entries since revision 4.05.

The table is split into three sectors; The first section defines two byte

"words" giving 16 bits of interface data; The second set is a set of jumps to

various functions, and finally the third set a set of pure data bytes.

1.9.1. Interface Data.

LNKFLG  Must be first entry in overlay at overlay address.  Is a two

byte address giving the size of the linkage table.  This is

used to check for consistency of overlay's

ENTSIZE Length of entry table, also used for consistency checking after

the overlay.  Currently 6

SYSEDT  The address of a dollar-terminated string giving the overlay

revision level and date.  Points to a string like:

CPXSYS.ASM(33) 4-JUN-1986$

FAMILY  The address of a dollar-terminated string giving the Family

overlay revision level and date.  If the system is in

CPXSYS.ASM rather than a particular Family overlay, it is

simply a pointer to $

1.9.2. Jump Table.

This is split into three main sectors-

1. Input/Output routines

2. Screen formatting routines

3. other system dependent routines

SELMDM

Parameters    None

Returns       None

Description   selects the modem port.  Most systems do nothing and simply

return.  HL,DE and BC registers preserved.

OUTMDM

Parameters    None

Returns       None

Description   Output the character in E register to the communications line.

BC,DE,HL registers preserved.

INPMDM

Parameters    None

Returns       Accumulator either 0 or character from comms line if available

Description   Check modem for character and if so, return it in A. HL,DE,BC

registers preserved, flags and accumulator lost.

FLSMDM

Parameters    None

Returns       None

Description   Clear any pending characters in the input buffer from the

modem.  No registers are preserved.

SELCON

Parameters    None

Returns       None

Description   Select the console.  This is a null subroutine for most

systems, but for IOBYTE systems selects the console.

OUTCON

Parameters    Character in E

Returns       None

Description   Send the character in E to the console.  Any quirks of system

responding in an odd manner should be handled.  No registers

preserved.

INPCON

Parameters    None

Returns       Zero or character in A.

Description   Get a character from the console or return a null if no

character to be read.  No registers are preserved.

OUTLPT

Parameters    Character in E

Returns       None

Description   Send the character in E to the printer.  The console is

selected.  Only DE registers are preserved

LPTSTAT

Parameters    None

Returns       00H or 0FFH in A register

Description   Test the printer to see if it is ready to receive a character

to be printed.  If a 00H is returned then the printer is ready

to receive a character.

EXTTER

Parameters    Character to be sent to the user supplied terminal emulator in

the E register

Returns       None

Description   If the user has supplied a terminal emulator in the overlay

code, EXTTER will be a JMP <non zero address>.  If SET TERMINAL

EXTERNAL has been set, all caharcters will be passed verbatim

to this terminal emulator.  If there is no external emulator,

this code will never be called.  The user should reset terminal

conditions on initialisation of both the system and before

CONNECT.  All registers should be preserved.

XBDOS

Parameters    Any required for calling BDOS

Returns       Any expected from the called BDOS routine

Description   This is an alternative entry to BDOS.  This entry will also

check the printer status etc.  For full details see the code

for the BDOS trap in CPSUTL.ASM.

2b)

CLRLIN

Parameters    None

Returns       None

Description   Clear the current line on the terminal

CLRSPC

Parameters    None

Returns       None

Description   Erase the current position (after a backspace)

DELCHR

Parameters    None

Returns       None

Description   Make delete (7FH) look like a backspace.  Some systems do a

backspace, space, backspace automatically others have to

simulate it

CLRTOP

Parameters    None

Returns       None

Description   Clear the screen and place the cursor at the top LH corner

SCREND

Parameters    None

Returns       None

Description   Place the cursor on the line for the Kermit-80 prompt after a

file transfer.  (Usually line 13)

SCRERR

Parameters    None

Returns       None

Description   Move cursor to the error message field on the file transfer

format screen

SCRFLN

Parameters    None

Returns       None

Description   Move the cursor to the filename field

SCRNP

Parameters    None

Returns       None

Description   Move the cursor to the packet count field

SCRNRT

Parameters    None

Returns       None

Description   Move cursor to the retry count field

SCRST

Parameters    None

Returns       None

Description   Move cursor to the status field

RPPOS

Parameters    None

Returns       None

Description   Move to the receive packet field (debugging use)

SPPOS

Parameters    None

Returns       None

Description   Move to the send packet field (for debugging use)

2c)

SYSINIT

Parameters    None

Returns       None

Description   Initialize the system specific items.  No registers are

preserved.  Any initialization is done once only when Kermit-80

is first loaded.

SYSEXIT

Parameters    None

Returns       None

Description   Program termination.  De-initialize anything in preparation for

a return to CP/M

SYSCON

Parameters    None

Returns       None

Description   Initialize anything before entering the connect state.

SYSCLS

Parameters    None

Returns       None

Description   System dependent close routine when exiting connect state

SYSINH

Parameters    None

Returns       None

Description   Help routine to test for any extensions to the escape menu

during the connect state.  If a system has any special feature

it can use during connect mode, then it can be tested as

<escape-character>xxx.  This entry is a string for printing to

the console for an <escape-character>?  Often used for

generating breaks or controlling a modem.

SYSINT

Parameters    None

Returns       None

Description   This is a test-and-jump on receipt of an escape sequence not

understood by Kermit-80.  If the character in A is not

recognized by your version of Kermit=80, do a rskip

SYSFLT

Parameters    Character in E

Returns       Character in E. Either a 00H or anything else in A

Description   Test the character in E. If it may not be printed to the

console, set A to zero.  All other registers preserved.

NB <XON>,<XOFF>,<DEL>,<NULL> are always rejected.

SYSBYE

Parameters    None

Returns       None

Description   System dependent processing for the BYE command.  (eg hang up

the phone)

SYSSPD

Parameters    Value from table in DE

Returns       None

Description   The system dependent code for baud rate change.  DE contains

the two byte value from the baud rate table.  This value is

also stored in "SPEED"

SYSPRT

Parameters    Value in DE

Returns       None

Description   The system dependent code for setting the port.  The parameters

are passed in DE, which are obtained from the port tables

SYSSCR

Parameters    String pointer in DE

Returns       None

Description   Setup the screen display for file transfer.  The Kermit version

string is pointed to by DE.  If the terminal is not capable of

cursor addressing (eg dumb glass TTY) then only the screen is

cleared and the version string is printed.

CSRPOS

Parameters    Row number in B, column number in C

Returns       None

Description   Move the cursor to row B, column C where B=1,C=1 is top LH

corner of screen.  The routine should first end a "cursor

position" leading string (up to four characters) then use the

parameters given to complete the versions cursor position

function

SYSSPC

Parameters    None

Returns       K bytes free in HL

Description   Get the amount of free disk space on the selected disk drive.

This could be in the system independent code.  Automatically

detects CP/M V2.2 or V3.0.  No registers saved.

MOVER

Parameters    Source Pointer in HL

Destination Pointer in DE

Byte count in BC

Returns       None

Description   Move (BC) bytes from (HL) to (DE) Z80 based systems do an LDIR,

while 8080 systems do it as a loop.  All registers destroyed

PRTSTR

Parameters    $ terminated string pointed to by DE

Returns       None

Description   Print the string onto the console.

3)

PTTAB         WORD      Points to VT52 equivalent escape sequences.

SPDTAB        WORD      Address of baud-rate command table, or 0 if table

does not exist

SPDHLP        WORD      Address of baud-rate help table, or 0 if SET SPEED is

not supported.

PRTTAB        WORD      Address of port command table or 0 if SET PORT is not

supported.

PRTHLP        WORD      Address of port help table or 0 if SET PORT is not

supported

TIMOUT        BYTE      FUZZY-TIMER.  Set to value suitable to your system

(depends largely on CPU speed)

VTFLG         BYTE      VT52 emulation flag.  Set to 0 if terminal emulates a

VT52, 01 if emulation is required, or 0FFH if emulations not

possible (eg for "CRT")

ESCCHR        BYTE      default escape character-usually control-] but

sometimes control-\

SPEED         WORD      Storage space for baud-rate.  Set to 0FFFFH as baud

rates are initially unknown.  Note that the STATUS routine only

looks at the first (least significant) byte.

PORT          WORD      Storage space for port.  Set to 0FFFFH as ports may

not be implemented, and is initially unknown

PRNFLG        BYTE      Printer copy flag-if O no copy.  Anything else =>

copy to printer

DBGFLG        BYTE      Debugging flag.  If O then no debugging to be done.

(ie writing of debugging info during a file transfer)

ECOFLG        BYTE      Local ECHO flag (default is off)

FLWFLG        BYTE      File warning flag.  If set to 1 will not overwrite

files already existing on disk with some-named files being

transferred

IBMFLG        BYTE      IBM system is the host-assume IBM file transfers etc

CPMFLG        BYTE      Flag indicating type of CP/M files to be transferred.

Default setting - DEFAULT

PARITY        BYTE      Type of parity in use

0 = Even parity

3 = Mark parity

6 = No parity (8th bit is data)

9 = Odd parity

12 = Space parity

SPSIZ         BYTE      Size of send packet

RPSIZ         BYTE      Size of receive packet

STIME         BYTE      Send timer (time-out)

RTIME         BYTE      Receive timer (time-out)

SPAD          BYTE      Send Padding (default=0)

RPAD          BYTE      Receive Padding (default=0)

SPADCH        BYTE      Send Padding character (default=NULL)

RPADCH        BYTE      Receive Padding character (default=NULC)

SEOL          BYTE      Send EOL character (default=CR)

REOL          BYTE      Receive EOL character (default=CR)

SQUOTE        BYTE      Send quote character (default=#)

RQUOTE        BYTE      Receive quote character (default=#)

CHKTYP        BYTE      Ascii value of checktype

31H="1"=checktype1 (6bits)

32H="2"=checktype2 (12bits)

33H="3"=CCITT checksum (CRC)

Default is 31H("1")

TACFLG        BYTE      If set to on (non zero) send the TACCHR twice.  This

is for ARPA TAC users, where the TAC swallows one "wakeup"

character.  If sent twice the TAC will pas one on and go back

to normal mode.

TACCHR        BYTE      Desired TAC character.  It is ignored if TAC trapping

is turned off. Value to send twice if TAC interception is set

on.  Default=0, but set to commercial AT if the conditional

assembly flag TAC is set true

BUFADR        WORD      Address of Multi-Sector buffering for I/O

BUFSEC        BYTE      The number of bytes the big buffers can hold.

Default is 1. (0=256 sectors).

FFUSSY        BYTE      Indicates if funny characters may be used in CP/M

file names (eg <>.,;?#[]) If zero, allow anything.  Default is

nonzero.

BMAX          SPACE:(2bytes) Highest block number on selected disk drive

BMASK         SPACE:(1byte) (Records/block)-1

BSHIFTF       SPACE:(1byte) Number of shifts to multiply by rec.block

NNAMS         SPACE:(1byte) Counter for file-names per line

1.10. Future Work

Work that needs to be done in future releases includes:

- Merge in support for additional CP/M-80 systems, particularly those

for which support was recently added to the monolithic v3.x source.

- Break up CPXSYS into discrete source files, one for each system.

These source files should serve as simple models for adding support

for new systems to Kermit-80 -- only the very basic screen

definitions, flags, i/o primitives, initializations, and so forth

should appear in each system-dependent file.

- Addition of missing features -- compression of repeated characters

during packet transmission, transmission of file attributes

(particularly size, so that "percent done" can be displayed for both

incoming and outbound files), command macros, more advanced login

scripts, remote operation and server mode, etc etc.  Any offers??

Index (Page numbers are approximate)...

8080   5, 6

Append   2

ARPAnet   10

Attention Character   10

Autoreceive   3

Baud   4

BIOS   5

Block Check   3

Bootstrapping CP/M Kermit   8

Break   2

Buffer size   3

Bye   2

Carriage Return   2

COLLISION   3

Connect   2

Control-C   2

Control-X   2

Control-Z   2

Copy   2

CP/M   6

CP/M-80 Kermit   1

CR   2

Debug   3

Default Disk   3

Directory   2

Directory file size   3

Downloading   8

Eighth-Bit Prefix   4

Erase   2

Escape Character   2, 3

Exit   2

External Terminal Emulation

4

File Copying   2

File-mode   3

File-Warning   3

FINISH   2

Flow Control   4

Generic Kermit-80   5

GET   2

Help   2

IBM   4

Input   2

Intercept Character   10

Local   2

Local-Echo   4

LOG   2

Logging   4

LOGOUT   3

NAK   2

No-exit   4

OUTPUT   3, 4

Pad character   4

Parity   4

Pause   3

Port   4

Print   3

PRINTER   4

RECEIVE   3

REMOTE   3

SEND   3

SET   3

Set padding   4

Set Receive   4

Set Send   4

Set Start of packet   4

Show   4

Status   4

TAC   10

TacTrap   4

Take   4

Terminal Emulation   4

Timeout   2, 7

TIMER   4

TRANSMIT   5

Type   5

USER   4

VERSION   5

Virtual Terminal   2

VT100 Emulation   4

VT52 Emulation   4

Warning   4

XON/XOFF   2

Z80   6

Table of Contents (page numbers are approximate)

1. CP/M-80 KERMIT                                                             1

1.1. Credits                                                               1

1.2. What's New                                                            1

1.3. Overview of Kermit Operation                                          1

1.4. Summary of CP/M                                                       1

1.5. Kermit-80 Description                                                 2

1.6. Kermit-80 Flavors                                                     5

1.6.1. Generic Kermit-80                                               5

1.6.2. CP/M 3 Kermit                                                   5

1.6.3. System-Specific Versions                                        5

1.7. Installation of Kermit-80                                             6

1.7.1. Organization of Kermit-80                                       7

1.7.2. Downloading Kermit-80                                           8

1.7.3. Assembling Kermit-80 from the sources                           9

1.8. Adding Support For A New System                                      10

1.9. Notes on New Features in Kermit-80 Version 4                         10

1.9.1. Interface Data.                                                10

1.9.2. Jump Table.                                                    10

1.10. Future Work                                                         12

Index                                                                        13

List of Figures

Figure 1-1:   Bootstrap program for Kermit-80 and CP/M Version 2.2         8

List of Tables

Table 1-1:   Kermit-80 SET PORT Options                                    5

Table 1-2:   Systems supported by Kermit-80 (Part 1)                       7

Table 1-3:   Systems supported by Kermit-80 (Part 2)                       7

Table 1-4:   Terminals supported by Kermit-80                              8

Table 1-5:   Terminals known to Kermit-80                                 10