HH     HH  PPPPPPPP    8888888    6666666

HH     HH  PP     PP  88     88  66

HH     HH  PP     PP  88     88  66

HHHHHHHHH  PPPPPPPP    8888888   66666666

HH     HH  PP         88     88  66     66

HH     HH  PP         88     88  66     66

HH     HH  PP          8888888    6666666

KK    KKK  EEEEEEEE  RRRRRRRR    MMM     MMM  IIIIII  TTTTTTTTTT

KK   KKK   EE        RR     RR   MMMM   MMMM    II        TT

KK  KKK    EE        RR     RR   MM MM MM MM    II        TT

KKKKK      EEEEEEE   RRRRRRRR    MM  MMM  MM    II        TT

KKKKKK     EE        RR RRR      MM   M   MM    II        TT

KK  KKK    EE        RR   RRR    MM       MM    II        TT

KK    KKK  EEEEEEEE  RR     RRR  MM       MM  IIIIII      TT

- 1 -

======================

HP86 KERMIT USER GUIDE

======================

Edition 1.00

This Manual applies to version 1.00

CONTENTS

--------

1. Introduction.

2. What is Kermit.

3. Running Kermit on the HP86.

3.1 Starting the HP86 Kermit.

3.2 Entering Kermit commands.

3.3 HP86 Kermit commands.

3.4 Using Kermit.

3.5 Finishing Kermit.

4. Communicating with another computer.

4.1 Setting the RS232.

4.2 Using the HP86 as a terminal.

4.2.1 The CONNECT command.

4.2.2 Differences between command mode and terminal mode.

4.2.3 Sending a break.

4.2.4 Returning to the HP86 Kermit.

5. Transferring files using Kermit.

5.1 The basic method.

5.2 File types.

5.2.1 The file type problem.

5.2.2 The data type problem.

5.3 HP86 to remote file transfer - The SEND command.

5.4 Remote to HP86 file transfer - The RECEIVE command.

5.5 Transfer problems.

5.6 File name conversion.

5.6.1 HP86 to standard conversion.

5.6.2 Standard to HP86 conversion.

5.6.3 File renaming.

6. Changing Kermit attributes.

6.1 The SET command.

6.2 The SHOW command.

6.3 Settable attributes.

Appendix A - HP86 Kermit capabilities.

Appendix B - HP86 Kermit error messages.

Appendix C - Converting numeric variables.

- 2 -

1. INTRODUCTION

===============

Program:       Martin Rootes, Sheffield City Polytechnic, U.K.

Language:      HP86 Basic.

Documentation: Martin Rootes.

Version:       1.00

Date:          April 1986

This manual describes the Kermit program for the HP86 microcomputer

produced  by  the  Computer  Services  Department  of  Sheffield  City

Polytechnic.       The   implementation   described   is   HP86 Kermit

version 1.00. The purpose of this manual is to supply  the  user  with

the information necessary to enable them to transfer files from a HP86

microcomputer to another Kermit.  As transfer will almost always be to

a  different  system the user will also have to read the corresponding

guide for the other Kermit.

This manual has been divided into  various  chapters,  dealing  with

various  aspects of the HP86 Kermit. For users new to Kermit Chapter 2

describes what Kermit is, the way it works, and why it is better  than

some  other  methods  of  file  transfer.    Chapter 3 details the way

HP86 Kermit is run, how to enter commands, brief descriptions  of  the

commands,  and  how  they are used.   Chapter 4 deals with setting the

HP86 Kermit up to communicate with another computer, and how to use it

as a  terminal  for  a  mainframe.    Chapter  5  gives  comprehensive

information on how to transfer files, and the problems associated with

transfer.    Following these are various chapters giving more detailed

information on commands and some hints and tips.

The HP86 Kermit version 1.00 is written in HP86 BASIC and needs  the

binary  program UTIL/1 to be loaded to provide various extra commands,

this program is provided with the HP86 microcomputer on the demo disk.

HP86 Kermit version 1.00 was developed, written  and  tested  on  an

Hewlett  Packard  HP86 microcomputer with a HP 82939A Serial interface

and the I/O programming ROM installed. Due to being written  in  BASIC

it  will  very probably work with the HP87 as long as the same I/O ROM

and interface are fitted, however this has not as yet been tested.

- 3 -

2. WHAT IS KERMIT

==================

Kermit is a data transfer system devised by the Center for Computing

Activities at the University of  Columbia,  New  York  (CUCCA).    The

copyright  on Kermit is retained by CUCCA, however they have published

full information on it and permit anyone  to  implement  it  on  their

machines, as long as this is not for commercial purposes.  Due to this

policy  Kermit  is now available for a large range of computers: There

are  implementations  for  nearly  all  mainframes  and  most  popular

microcomputers.

The  basic idea behind Kermit is to allow the transfer of any data

between different systems, in a uniform manner, and producing a usable

file on the receiving system. There are other methods of data transfer

that could be used, most notably 'dumb capture',  however  Kermit  has

several advantages over this method.

Most  simple  terminal emulator programs will allow some degree of

data transfer in the form of 'dumb capture'.  This is a method whereby

as characters are received from the mainframe they  are  placed  in  a

file, as well as being displayed on screen.  The reverse process being

to  take  characters  from a file on the microcomputer, instead of the

keyboard, and transmit them to the mainframe.

The dumb capture method, although fast and easy  to  implement,  has

many  problems  associated  with  it.  These  are  due  to  the  RS232

communications system used to link the computers, and  the  manner  in

which it is used.

The first problem is caused by parity, as many mainframe systems use

one bit of a byte for error checking, the 'parity' bit, only the lower

7  bits  of  a byte can be used to transmit data, thus only characters

with codes between 0 and 127 will be able to be sent.

Secondly communications systems commonly use certain  characters  as

control  characters,  these  can  therefore not be sent as part of the

data without causing problems, control codes lie in the range 0 to  31

and sometimes include 127.

Thirdly  in  a dumb capture system no error checking is performed,

this means that the file received may not necessarily be the  same  as

the  file  sent,  due  to  transmission  faults  as  well as the above

problems.

- 4 -

Kermit  overcomes  these  problems  by  encoding  the  data  to  be

transferred  using a standard set of rules, the Kermit 'protocol'. The

rules have been designed  to  allow  transmission  of  any  character,

regardless  of  any  parity  or  control  codes,  with  adequate error

checking to ensure that the file received is  the  same  as  the  file

sent.

The full Kermit protocol is beyond the scope of this manual, however

the basic method will be described here to aid in the comprehension of

following  discussions,  if more detail is required consult the Kermit

protocol manual (5th edition).

Kermit transfers data by dividing data up into  individual  packets,

each  packet  containing  the  data  itself  plus  some  extra control

information.  This information is used by the receiving Kermit to give

the start of packet, the sequence number of the packet, the length  of

packet,  the  type  of  packet  and  finally  a check byte. Using this

information the receiving Kermit can verify the  data  and  check  for

missing packets.

If  a  character needs to be sent whose code falls into one of the

problem areas as mentioned above (0-32, 128-255), Kermit will  convert

it  into  a  sequence of two 'legal' characters using prefix encoding.

This is achieved in the following manner, if  for  example  a  control

code  needs  to be sent as data it is first converted into a printable

character, corresponding with the  standard  representation  of  these

codes,  for  instance ASCII 13, usually called control M, is converted

to the letter M. This is  then  prefixed  with  the  prefix  character

(default - #), thus ASCII 13 will be sent as '#M'. A similar method is

used  to  encode  data  with  the 8th bit set using the 8th bit prefix

(default - &).

After sending a packet Kermit will  wait  for  a  packet  from  the

receiving  Kermit  to  confirm  that the packet has arrived correctly,

this is called the acknowledge packet (ACK). If  for  any  reason  the

packet is not received correctly the receiving Kermit sends a negative

acknowledge  (NAK),  this  causes  the  sender  to retransmit the last

packet. Thus if transmission errors occur Kermit will not only  detect

the error it will try to remedy the situation by repeating the data.

Although ensuring safe transmission of data this method can, without

adequate  safeguards, cause the system to become 'hung', however there

two mechanisms employed to stop this, timeouts and the retry limit.

The purpose of the timeout is to avoid the situation where  the  two

Kermits  are  each waiting for a packet from the other, this can occur

when a packet is lost in transmission. A timeout is basically  a  time

limit  on  the  receiving  of packets, if after the limit has passed a

packet has still not been received the Kermit  will  assume  that  the

last one it had sent was lost and will retransmit it.

- 5 -

The  retry limit is, as its name implies, a limit on the number of

times that Kermit will try to retransmit a  particular  packet  before

giving  up.  This  is to prevent the Kermit from endlessly sending the

same packet to a Kermit that is either  not  replying,  or  is  having

severe transmission problems.

The result of using the Kermit protocol is a simple, highly reliable

system  of  data transfer, that can be, and has been, implemented on a

wide variety of machines ranging from  microcomputers  to  mainframes.

The  only  requirements  needed  to  transfer  data  from the HP86 and

another computer are two Kermit programs, one for the HP86 and one for

the target computer, and the existence of  a  RS232  serial  interface

port  on  both  machines.  The  great  advantage  of  Kermit over data

transfer  programs  written  for  specific  machines   is   its   wide

availability,  making  it  possible to transfer data to many different

machines using the same program.

- 6 -

3. RUNNING KERMIT ON THE HP86

=============================

This section describes the general operation of the HP86 Kermit, how

to start it and how to enter commands.

3.1 STARTING THE HP86 KERMIT

----------------------------

To run Kermit on the HP86 the disk containing the Kermit program and

the utility programs should be inserted in the  disk  drive  (if  your

HP86  has  more  than one disk drive insert in drive 0). The following

procedure should now be taken:-

LOAD "Kermit"      <--- Load the Kermit program.

LOADBIN "UTIL/1"   <--- Load the utility program.

<INIT>             <--- Press the INIT button.

<RUN>              <--- Press the RUN button.

After about 80 secs the program will display the following prompt:-

Kermit-HP86 > Enter command ?

If this does not happen the RUN button should be pressed again.

3.2 ENTERING KERMIT COMMANDS

----------------------------

To control the HP86 Kermit commands have to be issued to it, this is

done by entering the desired  command  in  the  space  underneath  the

Kermit  prompt (note: on most implementations of Kermit the command is

entered next to the prompt).  Commands can be entered in either  upper

or  lower  case,  and  may  be abbreviated to a minimum of 1 character

(depending on command).

Some of the commands will require parameters to be entered as  well,

these  should be typed after the command and should be separated by at

least one space, except for filenames which must be enclosed by quotes

(").  The parameters can be of various types, a filename, a character,

a numeric value, or an item from a  list.  In  the  case  of  the  SET

command  two  sets  of parameters are required, firstly the item to be

set, and secondly what it is to be set to.

- 7 -

The question mark (?) can be used  to  give  useful  information  on

commands,  if  entered  as  a  command  all the legal commands will be

displayed, and if entered as a parameter the type of parameter will be

displayed.

send ?                                           <- You enter.

SEND "Source filename" <"Destination filespec">  <- Kermit replies.

Where '<' and '>' denote an optional item.

If the required parameter is an item from a list the alternatives are

all displayed, also note that list items can  be  abbreviated  in  the

same way as commands.

set flow ?                                       <- You enter.

FLOW-CONTROL - NONE, XON/XOFF, DTR/RTS           <- Kermit replies.

3.3 HP86 KERMIT COMMANDS

------------------------

The following is a list of valid commands for the HP86 with a brief

description of each, they will be explained in more detail  in  latter

chapters.

Command  Parameters  Description

CONNECT               Sets the  HP86  to act as a  terminal to allow

communication with the remote host.

SEND     "source filename" <"dest filespec">

Send  the  specified file  to the remote,  the

file name that is  used by the remote can also

be specified, if required to be different from

the source file name.

RECEIVE  <"dest filename">

Receive  a file,  or group of files,  from the

remote, changing filename if required.

SET      List option  Parameter

Set specified option  to a new value,  used to

change communication settings, timeouts, retry

limit etc. Requires extra parameter(s) after

the option.

SHOW     List option  Displays  current  setting  of  the  requested

option,  if  no  option  is specified  all the

settable parameters will be displayed.

EXIT                  Both these commands will cause the HP86 Kermit

QUIT                  to stop,  and return the  HP86  to the command

state.

- 8 -

CAT                   This command will  produce a  catalogue of the

current disk.

As  well as the above commands it is also possible to use the HP86

filing system commands in the following way:

<PAUSE>                       <- Press the PAUSE key.

Warning 101 at line 0 : XFR   <- The HP86  gives  a warning  that an

input transfer  from the  RS232  is

still in progress.

Any of the HP86 filing system  commands  can  now  be  entered,  for

example:

PURGE "GARBAGE"               <- Erase the file called "GARBAGE"

Once  finished  entering  commands  to the HP86 the Kermit program

should be restarted:

<CONT>                        <- Press the continue key

?                             <- The  HP86  replies  with this,  the

next  Kermit command  should now be

entered under this prompt.

3.4 USING KERMIT

----------------

To use Kermit to transfer data you will need a  Kermit  program  for

both  the HP86 and the other computer, the actual procedure to be used

depends on whether the other computer is a microcomputer with its  own

keyboard,  or  a  minicomputer  or  mainframe that needs a terminal to

communicate with it.

For HP86 to microcomputer transfer the method is very simple,  after

making  any  connections  load  and  run  the  Kermit programs in both

computers, then using the SET command the RS232 interfaces  should  be

set  to  appropriate values (see chapter 4). Once all the settings are

correct transfer can begin, enter the RECEIVE command on the  computer

to  which  the  data is to be sent, then enter the SEND command on the

sending computer, the transfer will now take place.

For HP86 to minicomputer or mainframe transfer a slightly  different

approach  is  needed,  after  physically  connecting  the  HP86 to the

computer the HP86 Kermit is run and any communication  settings  made.

The  HP86  should  now  be  put  into terminal emulator mode using the

CONNECT command, then using the HP86 as a terminal you should  log  on

to the computer and run its Kermit program. To transfer data you enter

the  required  command  on  the  remote,  either SEND or RECEIVE, then

escape back to the HP86 Kermit (using k1 - see chapter 4),  and  issue

the complimentary command.

- 9 -

After  the transfer is completed connect back to the remote, if no

more transfers are required exit from the  Kermit  program,  log  off,

escape back to the HP86 and exit from its Kermit program.

Both  these  methods will be discussed in more detail in chapter 5

'Transferring files by Kermit'.

3.5 FINISHING KERMIT

--------------------

Once you have finished using Kermit  for  your  file  transfers  you

should  exit  from  the  program  by using either the EXIT or the QUIT

command, either of these  will  halt  any  RS232  transfers  that  are

active,  close  any  open files and leave the HP86 in a suitable state

for further use.  If you wish to enter a new program via the  keyboard

you  should use the SCRATCH command to clear the memory, as the Kermit

program is still present.

- 10 -

4. COMMUNICATING WITH ANOTHER COMPUTER

======================================

This section describes how to set up the HP86 Kermit so that it  can

communicate  with  another  computer,  the  remote  host. it will also

outline how to use the terminal emulation facility when the remote  is

a mainframe computer.

4.1 SETTING THE RS232

---------------------

Once  the  HP86  is physically connected to the remote (if you are

unsure of how to do this contact your system support  personnel),  you

should  now set up the RS232 interface so that it can communicate with

the remote host. There are four separate settings that may have to  be

changed  these  are  Baud  rate,  parity,  duplex  and flow control or

handshake (depending on duplex setting).  All  the  changes  are  made

using  the  SET  command  and  the existing settings can be determined

using the SHOW command.

BAUD RATE     110, 300                           Default 300

This is the rate that the data is transmitted along the RS232  link,

at present only two rates are allowed 110 or 300 baud.

PARITY        NONE, ODD, EVEN, MARK, SPACE       Default MARK

This  is the parity that will be used by the RS232 interface, this

can be none, odd, even, mark or space, if none is chosen all 8 bits of

a byte will be sent, otherwise only the bottom 7 are used.

DUPLEX        FULL, HALF                         Default HALF

This defines whether the remotes communication equipment can receive

at the same time as it is transmitting, full duplex operation, or  can

only  receive  whilst not transmitting, half duplex. Setting this will

also change the local echo setting, if set to full duplex  local  echo

will  be  switched  off, is set to half duplex it will be switched on,

this can be changed with SET LOCAL-ECHO.

FLOW-CONTROL  NONE, XON/XOFF, DTR/RTS            Default NONE

Defines the method the remote uses to stop the HP86 transmitting any

more data, to stop its buffers overflowing. The possible settings  are

none, XON/XOFF for software flow control using control codes 17 and 19

(XON,XOFF),  and  DTR/RTS  for hardware flow control using the DTR and

RTS control lines. Setting this to any setting other  than  none  will

set handshake to none.

- 11 -

HANDSHAKE     NONE, BELL, LF, CR, XON, XOFF      Default XON

Defines the character to be used as a handshake character for half

duplex  communications,  the  handshake  character  is  used  by  both

computers  to  indicate  that they have finished transmitting and that

the other  can  now  transmit,  this  is  sometimes  called  the  line

turnaround  character.   The possible option are none, bell (ASCII 7),

linefeed (ASCII 10), carriage return (ASCII 13), XON  (ASCII  17)  and

XOFF (ASCII 19).

To change any of these settings use the SET command in the following

manner:

KERMIT-HP86 > Enter command ?  <- Underneath the prompt  type in set

set flow-control xon/xoff         followed by the options.

FLOW-CONTROL - XON/XOFF        <- Kermit  will respond with  the new

setting if it is accepted.

Note  that  the options can be abbreviated for ease of typing, for

instance the above could have been entered as set flow xon.

The settings can be determined using the SHOW command, if no  option

is  specified  the  screen  will  display  the settings of all the set

options, otherwise specify the particular option:

KERMIT-HP86 > Enter command ?

show parity                    <- Enter command

PARITY - MARK                  <- Kermit responds  with  the current

setting.

The actual settings needed will depend on the machine that you  wish

to communicate with, you will need to know what its settings are.

If you are linking to another microcomputer you will need to set the

RS232  options to the same on both machines. The simplest method is to

set the options on one machine to the default settings of  the  other,

depending  if  this can be done, for instance if the other micro has a

default Baud rate of 1200, which can not be set on  the  HP,  set  its

Baud rate to 300. An alternative approach is to set each option to the

'best'  that  can  be  set on both machines, ie the highest Baud rate,

full duplex, hardware flow control, and no parity (to allow 8 bit data

communication with no encoding).

When connecting to a mainframe computer you will be limited  to  the

standard operating on that computer, the default settings are designed

to  operate  with an IBM 4341 on a 300 baud line, your Systems Support

staff should be able to tell you what settings  are  needed  for  your

installation.

- 12 -

4.2 USING THE HP86 AS A TERMINAL

--------------------------------

As mentioned before when transferring files between the HP86 and a

minicomputer or mainframe you will need to communicate with the remote

by using the HP86 as a terminal, to achieve this you  should  use  the

CONNECT command.

4.2.1 THE CONNECT COMMAND

-------------------------

The  CONNECT command needs no parameters and can be abbreviated to

'C', once entered the HP86 will  clear  the  screen  and  display  the

following information:

HP86 Kermit - Terminal emulation mode

Function key   Escape character   Action

--------------------------------------------------

k1                 C           Return to Kermit

k7                 B           Transmit break

k14                            Enable transmit

The above display indicates the action of various function keys, and

the  corresponding  escape key sequences (Ctrl-] + character).  At the

bottom of the screen a cursor will be displayed,  this  is  where  any

keyboard input will appear.

The HP86 will now act like a 'glass' teletype, characters typed on

the keyboard will be  sent  directly  to  the  remote  via  the  RS232

interface, and characters received from the RS232 will be displayed on

the screen.

4.2.2 DIFFERENCES BETWEEN COMMAND MODE AND TERMINAL MODE

--------------------------------------------------------

There are important differences in the way the keyboard and screen

are handled in the terminal mode compared to  the  normal  HP86  entry

mode, these are as follows.

All of the special keys, cursor keys and screen control keys cease

to have any effect, except for the  ENTER  key,  RESET  key,  and  the

defined function keys.  Pressing the ENTER key sends a carriage return

(Control M)  to  the  remote,  in addition if a handshake character is

specified it will also be sent and the transmitter will  be  disabled.

The  RESET key performs its normal function of halting the program and

resetting interfaces.

- 13 -

The delete key may not have any  effect,  depending  on  the  remote

system, even if accepted the action on the screen will differ, instead

of  the  last  character typed being removed from the screen, a delete

character will be displayed  after  it.    Some  remotes  will  use  a

different  key  for  this  purpose, for instance the IBM uses '_' as a

delete character, you  will  be  able  to  find  this  from  the  user

documention on using terminals.

If  there is any handshake or flow control specified there will be

times when the keyboard does not respond due to the transmitter  being

disabled,  with  half duplex systems this will occur immediately after

pressing the enter key and  will  remain  until  the  remote  finishes

transmitting  data  and sends the handshake character. If the keyboard

is not responding and the remote is  not  transmitting,  or  about  to

transmit,  the  transmitter can be re-enabled by pressing function key

14.

4.2.3 SENDING A BREAK

---------------------

Many mainframe computers may need to be sent a  'Break'  signal  for

certain  reasons,  usually to interrupt the current program and return

to the command state. You can send a break from the HP86 Kermit either

by pressing function key 7, or by typing control ] followed by B.

4.2.4 RETURNING TO THE HP86 KERMIT

----------------------------------

When you have finished issuing the required commands to  the  remote

Kermit  or you have finished your session on the remote, you will need

to return to the HP86 Kermit's command mode, this  is  achieved  using

function key 1, or control ] plus 'C'. On returning the screen will be

cleared  and the standard Kermit prompt will be displayed.  If you are

unsure what mode  you  are  in  examine  the  prompt,  if  it  is  not

'KERMIT-HP86 > Enter command ?' then you are in terminal mode.

- 14 -

5 TRANFERRING FILES USING KERMIT

================================

The  main  purpose  of  the HP86 Kermit is to enable file transfer

between the HP86 and another computer,  the  remote  host.  The  basic

method  will not vary from one system to another as this is defined in

the Kermit  protocol,  however  certain  details  will  be  different.

Before  attempting any file transfer you should consult the user guide

for the remote system to familiarise yourself  with  any  differences.

This  section  describes  in  detail how to transfer files between the

HP86 and the remote computer.

5.1 THE BASIC METHOD

--------------------

To transfer files using Kermit you will need another Kermit  program

running in the remote host, and you will need to issue commands to it.

For  HP86 to microcomputer transfers this can be done simply by typing

the required commands on the respective keyboards, however for HP86 to

mainframes or minicomputers a more complex approach is needed.

1. Load and run the HP86 Kermit, change any RS232 settings to  enable

the  HP86  to  communicate  with  the  remote,  and  set any other

parameters needed for the transfer.

2. The HP86 is put into terminal mode using the CONNECT command.  Now

with the HP86 acting as a terminal the user should log on  to  the

mainframe in the normal manner.

3. The  Kermit  program  on the remote should now be run, and any SET

commands entered.

4. To transfer files the relevant  command  should  now  be  entered,

RECEIVE  for  HP86  to  remote,  SEND  for  remote  to HP86. After

entering the command return to HP86 Kermit command mode  using  k1

and  enter  the  complimentary  Kermit  command,  SEND for HP86 to

remote, RECEIVE for remote to HP86.

5. After the transfer is complete re-enter terminal mode to  set  the

remote's  Kermit for any further transfer, or if you have finished

all your transfers you should log out from the remote.

The above description is only a guide to the sequence of actions  to

be  taken,  you may not need to set any parameters, or you may wish to

run some other program on the remote using data transferred by Kermit.

- 15 -

5.2 FILE TYPES

--------------

Before any discussion of file transfer it is  necessary  to  outline

the   types  of  files  that  can  successfully  be  transferred  with

HP86 Kermit version 1.00.  Due  to  version 1.00  being   written   in

HP86 BASIC  the  ideal  of  being  able to transfer any file cannot be

implemented.  The restrictions imposed by HP86 BASIC are quite  severe

and  can only be overcome by various forms of file conversion. Because

of the limitations it is advisable to  thoroughly  read  this  section

before attempting any file transfer.

5.2.1 THE FILE TYPE PROBLEM

---------------------------

When a file is created by the HP86 it is assigned a particular type

depending on its use and method of generation, these are as follows:-

PROG - BASIC program created using the STORE command.

BPGM - Binary program.

DATA - Data file.

GRAF - Screen dump.

**** - Extended file.

Of these only those of type DATA can be sent using  version  1.00  of

Kermit,  this  being the only type that can be accessed by HP86 BASIC.

Fortunately a utility program 'GETSAVE' exists on the HP86  demo  disk

which  will  save  BASIC  programs from memory as DATA files, and load

DATA files as BASIC programs (or sections of programs).

The file type can be discovered using the CAT command either  before

running Kermit or whilst in the command mode.

KERMIT-HP86 > ?

cat                             <- Enter catalogue command

.. HP replies with

Name        Type  Bytes   Recs

------------------------------

MYFILE      DATA    256      4

PROGRAM     PROG    256      9

UTILITY     BPGM    256      7

If  the  type  displayed  is  not  DATA  then  the  file cannot be

transferred  using  HP86 Kermit version 1.00,  if  this  is  attempted

Kermit will return with an error.

- 16 -

5.2.2 THE DATA TYPE PROBLEM

---------------------------

As  previously mentioned only files of type DATA can be sent via the

present implementation of Kermit, however there  is  a  second  factor

that can affect the success of any transfer, that of data types.

The  HP86 like many other microcomputers stores data on its disks in

two different formats depending on the variable type,  either  numeric

or  string  data.  This  causes problems when sending data to a remote

kermit for the following reasons:

All data to be transferred via Kermit has to be either  ASCII  data,

or binary data that can be converted to ASCII by 8-bit prefixing.

HP86  BASIC  is unable to access data on disk as bytes, it must be

read either as a string variable or as a numeric variable.

Therefore if data in the numeric format is encountered it has to  be

converted into ASCII before it can be sent. There is a mechanism to do

this   in  the  HP86  Kermit  program,  however  this  method  is  not

satisfactory for most transfers for the following reasons:

The numeric variable will be converted to a  string  of  characters,

the  number  of which depends only on the number of significant digits

in the variable (for instance '1 ','10 ' and '10.04 '). The  resulting

file will therefore have a irregular format that will make its use, by

other computer systems or languages, very difficult.

There  is  no method in the Kermit program of converting this data

back  into  numeric  variables,  therefore  a  file  sent  using  this

conversion  and  then  received back will be of a different format and

almost definitely unusable.

For these reasons it is advisable to convert any files to be sent  by

kermit  so  that  any  numeric  variables  are stored as, or in, ASCII

strings.  You should attempt to do this so the numbers are stored in a

fixed format based on their range, this will enable  the  data  to  be

reconverted  easily,  and  allow  it  to  be used as input to programs

written in other languages, or on different computers.

For example if the data was a list of temperatures between 0 and  200

measured  with  and accuracy of 2 decimal places, a sensible choice of

format would be XDDD.DD (where D is a digit  or  space  if  a  leading

zero,  X  is a space), 10 of these numbers could then be placed in one

string, producing records with the following format:

12.15  54.72 112.39  23.54 178.01 107.10  93.42 121.34 154.81 133.04

This would produce a file that could be  used  by  many  high  level

languages,  for  instance  it could be read by a FORTRAN program using

the following format statement - FORMAT (10(1X,F6.2)).

- 17 -

For a simple method of storing numeric data as formatted strings see

appendix C.

Please note that there is no way that you can  determine  whether  a

file  contains  data of the correct type until you actually attempt to

transfer it. The default condition for the HP86 Kermit  is  to  do  no

conversion  of numeric data, if a numeric variable is encountered then

the transfer will be halted and an error message displayed. To use the

inbuilt conversion facility issue the following command:

SET SEND-CONVERT ON

It  must  be  emphasized  again  that  using  this   method   produces

unpredictable  results,  also  in some circumstances an type error can

still be produced.  For these reasons it is suggested that  it  should

be  used  vary rarely, for files containing only a small proportion of

non crucial numeric data.

5.3 HP86 TO REMOTE FILE TRANSFER - THE SEND COMMAND

---------------------------------------------------

To send a file from the HP86 to the remote Kermit you use  the  SEND

command,  however  before  you  issue  this command you should set the

remote in to receiving state. To set the remote up to receive  a  file

you  use the RECEIVE command, the manner in which this is done depends

on the type of remote.

If the remote is a  microcomputer  simply  enter  RECEIVE  using  its

keyboard then enter the SEND command using the HP86's keyboard.

For  a mainframe or minicomputer remote host you will have needed to

have logged on and started Kermit using terminal mode. While still  in

terminal  mode issue the RECEIVE command. You should now return to the

HP86 Kermit command mode  by  using  function  key  1  and  enter  the

HP86 Kermit SEND command.

Once the remote is in a state to receive files you should enter the

SEND command, this has the following format:

SEND "source filename" <"dest filespec">

Where:

"source filename"  Is the name  of the file to be  sent in  standard

HP86 format.

"dest filespec"    Is the name  that you wish the file  to be stored

as on the remote computer.

If only the source filename is specified the HP86 Kermit  will  send

this name converted into a 'legal' Kermit format.

- 18 -

The HP86 will now start to send the named file to the remote Kermit

the following screen display will be seen:

----------------------------------------------------------------

I HP86 Kermit sending file                                     I

I ------------------------                                     I

I                                                              I

I Sending 'myfile dat' as 'MYFILE.DAT'                         I

I                                                              I

I Current action : Sending initialise      Retries : 0         I

I Packets sent     : 0               NAKs received : 0         I

I Bytes sent       : 0k              Timeouts      : 0         I

I                                    Bad packets   : 0         I

I                                                              I

The file will now be sent packet by packet  to  the  remote  Kermit.

The  progress  of  the transfer will be displayed on the screen.  When

completed the message 'Transfer successful' will be displayed and  you

will be returned to command mode.

5.4 REMOTE TO HP86 TRANSFER - THE RECEIVE COMMAND

-------------------------------------------------

Before discussing the method used to transfer files from the remote

to the HP86 a serious problem needs to  be  highlighted.  Unlike  most

filing systems the HP86 BASIC filing system is incapable of creating a

file  without  knowing its size. Unfortunately there is no way for the

HP86 Kermit to know the size of the file it is receiving. Due to  this

problem  it  is necessary to inform the HP86 Kermit of the size of the

file that it is to receive, to do this use the SET FILE-SIZE command.

SET FILE-SIZE size

Where size is the desired  file  size  in  kilobytes,  the  default

setting is 10.

If  FILE-SIZE  is  not  set  correctly  one of two problems can arise.

Firstly if FILE-SIZE is set far larger than the actual size  you  will

be  wasting  disk space.   Secondly and more seriously if FILE-SIZE is

smaller than the actual size then the transfer will  be  aborted  when

the file on the HP86's disk becomes full.

To  avoid  both problems before receiving files find the actual file

size by using the relevant command on the remote system, you can  find

this  from the user documentation.  You may have to calculate the size

in kilobytes from the record size and number of  records,  round  this

figure  up  to  the nearest kilobyte.  Setting FILE-SIZE to this value

will usually be sufficient.  Occasionally though this can be less than

actually needed as the HP86 requires  an  extra  3  bytes  per  record

stored, so for added safety when transferring large files increase the

file size by 1.

- 19 -

As  you would expect, to receive files from the remote you use the

RECEIVE command on the HP86 Kermit and the SEND command on the remote.

The order the two commands are entered will depend again on  the  type

of remote.

For microcomputer to HP86 transfer: Issue the RECEIVE command on the

HP86, then the SEND command on the remote.

For  transfer from mainframe or minicomputer: First issue the SEND

command to the remote whilst in terminal mode,  then  return  to  HP86

command mode, and enter RECEIVE.

The RECEIVE command has the following format:

RECEIVE  <"dest filename">

Where "dest filename" is the file name that the received file will

be stored as, if not supplied the name sent by the remote Kermit  will

be converted to a legal HP86 name.

Once  entered the HP86 screen will clear and the following will be

displayed:

----------------------------------------------------------------

I HP86 Kermit receiving file                                   I

I --------------------------                                   I

I                                                              I

* I                                                              I

I                                                              I

I Current action : Wait for initialise     Retries : 0         I

I Packets received : 0               NAKs sent     : 0         I

I Bytes received   : 0k              Timeouts      : 0         I

I                                    Bad packets   : 0         I

I                                                              I

* Note the name of the file being received  will  not  be  displayed

until that information has been received from the remote.

When  the  HP86 Kermit receives the file header it will attempt to

create a file with either the name supplied after the RECEIVE command,

or with the name contained in the header (converted to HP86 form).  If

the file already exists on the disk then Kermit will rename  the  file

and attempt to store it again, the new name will be displayed.

Receiving 'MYFILE.DATA' as 'MYFI00 DAT'

The actual way the file is renamed depends on whether the name was

supplied as a parameter to the RECEIVE command, or  contained  in  the

file header, see the section on file name conversion for more details.

- 20 -

5.5 TRANSFER PROBLEMS

---------------------

After  the transfer has completed the HP86 Kermit will display the

'Transfer successful' message, however if an error has occured and the

transfer has been halted a suitable message will be displayed.

Timeout           -  Time limit exceeded   on sending or  receiving  a

packet, increase timeout and re-run.

NAK received      -  A  negative  acknowledge packet was received from

the remote that caused  the  retry  limit  to  be

exceeded.

Checksum error    -  A  garbled  packet  was received from the remote.

Check   communication   settings   and   physical

connections.

Incorrect  packet  -  The remote Kermit has sent a packet which is not

the type that the HP86 expects.  This should only

happen if both Kermits  have  been  set  to  SEND

files,  try  again  being  careful  to  enter the

correct commands.

Error message from remote

-  The remote Kermit has sent an  error  packet  the

contents of which will be displayed.

Transfer aborted  -  Function key 1 was pressed during transfer.

Other messages    -  The rest of the messages generated by Kermit will

be  errors  produced  by  the  filing  system for

instance 'File does not exist', these are  mostly

self   explanatory,   however  the  error  no  is

displayed so users can check the HP86 manual  for

more detailed descriptions (see also Appendix B).

Sometimes on connecting back to remote, especially after an error,

garbage is printed on the  screen.  This  indicates  that  the  remote

Kermit  is  still  in receive or send mode, having missed the relevant

error or break packet. To rectify this situation press the  enter  key

until the remotes prompt reappears.

5.6 FILE NAME CONVERSION

------------------------

The Kermit protocol specifies that the file name of the file being

sent should be transmitted in the file header packet, to  provide  the

receiving Kermit with a name to store the file under. However the file

name  has  first  to  be  converted  to  a  standard  format, although

different from the HP86 format it is very similar  to  many  mainframe

file specifications being of the following form:

filename.filetype

where filename and filetype can be of any length but can only be

composed of digits or uppercase letters.

- 21 -

HP86 file specifications are of the form "XXXXXXXXXX",

or "XXXXXXXXXX.volume",

or "XXXXXXXXXX:msus", where

XXXXXXXXXX      is a filename of up to 10 characters.

volume          is the volume label of the disk containing XXXXXXXXXX.

msus            is the device specifier of the disk drive containing

XXXXXXXXX (E.G. D700).

HP86 file names may contain upper or  lower  case  letters,  digits,

special  characters,  excluding period and colon, and imbedded spaces.

Upper and lower case letters are not equivalent.

Due the differences in the format of filenames it  is  necessary  to

convert  names  used on the HP86 if they are to be sent to the remote,

and likewise convert names received from the remote if they are to  be

used on the HP86.

5.6.1 HP86 to standard conversion

---------------------------------

When  sending a file to the remote and a destination file name has

not been specified the HP86 Kermit will convert  the  source  filename

into a 'legal' Kermit name.

The  way this conversion takes place is as follows:  All lowercase

characters are converted to uppercase.  If the name  contains  one  or

more  spaces  then  the  first space will be converted to '.', else if

name contains one or more '_' characters then the first  '_'  will  be

changed to '.'.  The name is now scanned for legal characters (0-9,A-Z

& first '.') if any illegal character is found it will be removed from

the  name.  If the resulting name contains no '.' the filetype '.DATA'

will be added to the end.

For example

Statistics  would be converted to  STATISTICS.DATA

TEST FILE     ''  ''    ''     ''  TEST.FILE

Tst File 1    ''  ''    ''     ''  TST.FILE1

TEST#1-00     ''  ''    ''     ''  TEST100.DATA

The name that Kermit sends to the remote will be  displayed  on  the

screen along with the source file name.

- 22 -

5.6.2 Standard to HP86 conversion

---------------------------------

When receiving files without specifying a destination file name then

a  reverse  conversion  occurs.  First the name is scanned for the '.'

marker, if this does not exist then the name is used as it  is.    The

HP86  name  is  then  constructed  using the first 6 characters of the

filename, a space and then the first 3  characters  of  the  filetype.

Thus:

STATISTICS.DATA  would be converted to  STATIS DAT

TEST.FILE          ''  ''    ''     ''  TEST FIL

Note that a filename converted to Kermit format and then converted

back will not necessarily be the same.

5.6.3 File renaming

-------------------

When receiving files from the remote the situation  can  arise  that

the  file  name  supplied,  or the converted file name from the remote

will clash with a file already on the disk drive. To avoid losing data

by overwriting the existing  file  the  HP86 Kermit  will  attempt  to

rename  the  file. The method used for renaming depends on whether the

name was supplied as a parameter for the RECEIVE command, or converted

from the file header sent by the remote.

For a supplied name the method is to add a two digit number  to  the

end,  keeping  the  total  length  to  10 characters, this may involve

writing over the last two characters.  The numbering will  start  from

00  and  will  continue  until 99 after which an error message will be

generated.

For example:-

MYFILE      would be renamed  MYFILE00

STATISTICS    ''  ''   ''     STATISTI00

If MYFILE & MYFILE00 existed then

MYFILE      would be renamed  MYFILE01

If the file name is one that has been received  and  converted  from

the  file header packet then the renumbering is done at the end of the

part that was the filename, keeping it to 6 characters or less.

For example

MYFILE.DATA is converted to MYFILE DAT and renamed MYFI00 DAT

TEST.BASIC  ''    ''     '' TEST BAS    ''   ''    TEST00 BAS

The result of either of these  methods  will  be  displayed  on  the

screen, however the results should not be too radically different from

the original to enable correct identification.

- 23 -

6. CHANGING KERMIT ATTRIBUTES

=============================

At various points throughout this manual it has been mentioned that

certain attributes have needed to be changed, the specific cases being

described. This chapter gives  a  more  detailed  description  of  the

command used to change these attributes, the SET command.

6.1 THE SET COMMAND

-------------------

The SET command has the following format:

SET option parameter(s)

Where

option        The attribute that you wish to change.

parameter(s)  The value(s) to set the option to.

The parameter types will depend on the option specified, and can be

a character, a numeric value, or an item from a list. The option  name

can be abbreviated, as can the parameter if it is a list item.  Kermit

will  respond  with  the  new  setting  or with an error message if an

illegal value is entered.

SET RETRY 7        <- Enter set command

RETRY - 7          <- Kermit replies

SET RETRY #        <- Set command with incorrect value.

Illegal value - #  <- Kermit replies.

If a question mark is entered instead of the parameter  then  Kermit

will display either the expected parameter type, numeric or character,

or display the list from which the parameter should be chosen.

SET RETRY ?                             <- Set command query.

Numeric value                           <- Kermit replies

SET FLOW ?                              <- Set command query.

FLOW-CONTROL - NONE, XON/XOFF, DTR/RTS  <- Kermit replies.

- 24 -

6.2 THE SHOW COMMAND

--------------------

This  command  is  used  to  display  the  current settings of the

attributes, if entered with no option  then  all  settable  attributes

will  be  displayed,  otherwise the setting of the specified option is

shown.

SHOW DEBUG   <- You enter.

DEBUG - OFF  <- Kermit responds.

6.3 SETTABLE ATTRIBUTES

-----------------------

There now follows a list of the  attributes  that  can  be  set  and

displayed using the SET and SHOW commands.

OPTION         PARAMETER TYPE      DEFAULT

TIMEOUT        Numeric value       Default - 10 secs

The  length  of  time  the  HP86 Kermit will wait for a

packet from the remote before causing a timeout.

RETRIES        Numeric value       Default - 10 retries

The number of times the HP86 Kermit will try to  resend

a packet.

SEND-CONVERT   OFF, ON             Default - OFF

Defines  whether  numeric variables are to be converted

and sent as string data, if off numeric variables  will

cause the transfer to abort.

DEBUG          OFF, ON             Default - OFF

If  set  to  on  then packets sent and received will be

displayed on the screen.

PREFIX         Character           Default - #

Defines the character to be used for prefix encoding.

END-OF-LINE    Numeric value       Default - 13

The ASCII value of  the  end  of  line  character  that

should be sent after each packet.

RECORD-END     Numeric value(s)    Default - 13 10

Defines   the   sequence  of  control  characters  that

indicate the end of a record. This  will  be  added  to

strings  when  sending  files, searched for and removed

when receiving files.

FILE-SIZE      Numeric value       Default - 10

- 25 -

The size of the file (kilobytes) that will  be  created

when receiving files.

RECORD-LENGTH  Numeric value       Default - 256

The  record  length  that will be used by the HP86 when

creating files, if matching record  lengths  on  remote

systems  add  3 bytes, the HP86 needs these extra bytes

when storing strings.

NO-OF-RECORDS  Numeric value       Default - 40

The number of  records  of  the  file  created  by  the

RECEIVE   command,   setting  this  will  override  the

file-size, the resulting size being the  record  length

times  the no of records, setting file-size will change

this value.

DUPLEX         FULL, HALF          Default - HALF

The duplex setting of the remote,  setting  this  value

alters  the setting of the local echo, FULL - OFF, HALF

- ON.

LOCAL-ECHO     OFF, ON             Default - ON

Defines whether characters typed on the  keyboard  will

be  displayed  on the screen (ON), this only applies to

terminal mode.

FLOW-CONTROL   NONE, XON/XOFF, DTR/RTS        Default - NONE

Specifies the  flow  control  method  to  be  employed,

setting   this  to  any  value  other  than  NONE  sets

HANDSHAKE to NONE.

HANDSHAKE      NONE, BELL, LF, CR, XON, XOFF  Default - XON

Defines the handshake character to  be  used  for  line

turnaround,  setting  this to anything except NONE will

set FLOW-CONTROL to NONE.

PARITY         NONE, ODD, EVEN, MARK, SPACE   Default - MARK

The parity to be used by the RS232 interface.

- 26 -

Appendix A : HP86 KERMIT Capabilities

=====================================

The following is a list of Kermit capabilities, with information  as

to whether they are supported by HP86 Kermit version 1.00.

Local operation:                   Yes

Remote operation:                  No

Transfers text files:              Yes

Transfers binary files:            No

Wildcard send:                     No

^X/^Z interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 No

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes

Communication settings:            Yes; duplex,  parity,  handshake,

flow control

Transmit BREAK:                    Yes

IBM communication:                 Yes; default settings

Transaction logging:               No

Session logging (raw download):    No

Raw upload:                        No

Act as server:                     No

Talk to server:                    No

Advanced commands for servers:     No

Local file management:             Yes; CAT

Handle file attributes:            No

Command/init files:                No

Printer control:                   No

- 27 -

Appendix B : HP86 error messages

================================

The following is a list of error messages that Kermit version 1.00

will give if a transfer is aborted.

Timeout receiving   The HP86 has timed out whilst waiting for a packet

from the remote.

NAK received        The remote has sent a NAK causing the retry  limit

to be exceeded.

Checksum error      The last packet received was garbled.

Incorrect packet    The  wrong  packet type has been received from the

remote.

Timeout sending     The HP86 has timed out whilst  trying  to  send  a

packet.

Cannot rename file  The  file  being  received has the same name as an

existing file, but it cannot  be  renamed  as  the

number suffix is past 99.

Disc write protected

The  disk  has a write protect label attached, the

file cannot be stored.

File does not exist The file cannot be found on the disk.

Incorrect file type The file cannot be sent as the file  type  is  not

DATA.

Read error          There has been a read error on the disk.

Record does not exist

The  file  has become full before all the data has

been received.

No M.S. device      No mass storage device has been specified.

Directory full      No more files can be stored on  the  disk  as  the

directory has become full.

Volume not found    The specified volume name can not be found.

MSUS not found      The specified mass storage device (disk drive) can

not be found.

Read verify error   The data written to disk has been corrupted.

- 28 -

Disc full           The  disk  has  become  full,  or does not contain

enough space to create a new file.

Medium damaged      The disk is faulty.

Disc drive fault    The disc drive is not functioning  correctly,  can

be  caused  by leaving the latch on the disc drive

up.

Data type error     The variable just read was a numeric variable  and

the data conversion facility is off.

Transfer aborted    Function key 7 was pressed aborting the transfer.

- 29 -

Appendix C : Converting numeric variables

=========================================

As  mentioned  in  the  section on file transfer numeric variables

cannot be sent to the remote without conversion. Although a very basic

conversion method is included  as  part  of  the  HP86 Kermit,  it  is

advisable  not to use it but to convert the whole file into one usable

by Kermit and the remote.

To convert the file you will need to know the manner  in  which  the

data  is  arranged  in the source file, and decide on a format for the

converted file. The structure  of  the  source  file  can  usually  be

determined  by  examining  the  program  that  created it, with a well

documented program this task should be fairly  easy.  However  if  the

creating  program  is not available, and the structure is unknown, use

the HP86 BASIC TYP command.

Once you have determined the structure  of  the  file(s),  and  have

decided on a logical format to convert them to, you will need to write

a  short  program  to  achieve this. The easiest way of converting the

numbers into a useful format is to use  a  facility  provided  by  the

HP86 I/O ROM.  The method involves defining a dummy I/O buffer, output

the  numeric  variable  to this buffer using the formatting facilities

(USING), and reading it back in as a string with the ENTER command.

For example

10 DIM dummy$"88"    <- Dimension  variable to  8 bytes  more than

required size.

20 IOBUFFER dummy$   <- Set it up as an I/O buffer

.

.

.

500 OUTPUT dummy$ USING "X,M4D.DD"; N  <- Output number  using the

specified format.

510 ENTER dummy$ USING "K" ; N$       <- Read it in as a string.

- 30 -

After converting the number to a string  it  can  either  be  stored

separately,  or placed into another string along with others to form a

record. The OUTPUT  command  can  be  used  to  construct  records  by

outputting  more  than  one variable at once, or by outputting without

end of line.

500 OUTPUT dummy$ USING "XM4D.DD" ; N1,N2,N3,N4

or

500 FOR I=1 TO 10

510 OUTPUT dummy$ USING "#,X,M4D.DD" ; N(I)  <- Output without EOL

520 NEXT I

.

.   As there is now no  EOL sequence  at the end of the buffer the

.   following statement would be needed to read this back.

.

530 ENTER dummy$ USING "#,#K" ; R$           <- Enter without EOL

The complete record string can now be written to the disk file.

As can be seen this is a very simple method of  producing  formatted

files which can be sent using the HP86 Kermit. Also with a bit of care

these  files  could  be  used  as  input  to programs written in other

languages than BASIC. The reverse conversion is also easy  to  perform

using the ENTER command:

500 OUTPUT dummy$ ; R$  <- Output the string to the dummy buffer

510 FOR I=1 T0 10

520 ENTER dummy$ USING "#,X,M4D.DD" ; N(I) <- Get number

530 NEXT I

All  the  above conversion techniques can of course be included in

other programs, if it is invisaged that the data files produced by the

program will be transferred to another system using Kermit.  Including

an option to produce files in 'Kermit' format is more  efficient  than

having separate conversion programs.

The  above  discussion  does  not  include  any  details  of image

specifiers and other related matters, if you wish to use  this  method

of  data  conversion  you  are strongly advised to read the section on

Formatted I/O operations in the HP86 I/O Programming guide.

- 31 -

#